Host Master II+ Addendum (version 3.3)

Page v (Table of Contents)

Add after "70 .......GTOR STANDBY"

70.......TOR Standby

Page 70 (Before "Return to Packet")

Add the following:

TOR Standby

KAM Plus or KAM with Enhancement Board version 7.1 or later (if

All Mode was chosen in Hostset)

This selection will put the KAM HF port in the TOR Standby mode.

From TOR Standby you can transmit and receive AMTOR FEC signals

(used to call CQ). The KAM will respond to a link request in

AMTOR, Pactor, or G-TOR. When a station links to you in any of

these modes, Host Master will operate as described in the AMTOR,

Pactor, or G-TOR sections of this manual. In summary, pressing

the INS key while you are the IRS will seize the link and

pressing the INS key while ISS will peform a changeover. To

disconnect from the other station, simply press F3.

In addition, you may initiate a link in G-TOR mode by pressing

the F2 key and entering the callsign of the station you wish to

call. When this link is terminated, the KAM will return to GTOR

STANDBY.

Page I4 (Index)

Add:

TOR 70