GMON, a G-TOR(tm) Monitoring Program

for PC Clones (286 at 16 Mhz or above.)

3-15-94

Copyright 1994 by Kantronics Co., Inc. All Rights Reserved.

G-TOR is a trademark of Kantronics Co., Inc.

For use with KAM Plus or KAM with Enhancement Board

G-TOR monitoring requires KAM/KAM Plus version 7.0 or later.

PURPOSE

GMON is a basic terminal program that includes G-TOR monitoring

capability. GMON is included on your PACTERM distribution

diskette (free of charge) specifically to allow you to monitor G-

TOR links between two other stations. It is not intended to be a

general purpose terminal program. PACTERM and GMON are basic and

not full-featured programs. To operate multi-mode and split

screen, we recommend Kantronics' Host Master Series of terminal

programs.

ABOUT THE DOCUMENTATION

We have assumed that you are somewhat familiar with your TNC,

your PC, typing DOS commands, loading and storing diskettes, etc.

This manual does not cover instruction in these items. For

information on G-TOR, refer to the Enhancement Board (for the

KAM) or KAM Plus documentation.

SYSTEM REQUIREMENTS

Due to the high level of computation required by GMON to monitor

G-TOR activity in real-time, your PC or clone must be a 286 (16

Mhz) or above. GMON.EXE requires a 286 computer or better to

run. For 8088 processors, we have also provided GMONX.EXE. In

some cases, GMONX may allow you to monitor data but not traces

on your 8088. Slower 8088 machines will not be able to monitor

data real-time, however you may use GMONX to capture data to

disk for later off-line monitoring. GMON (for 286 and above) may

also capture data to disk for off-line monitoring if desired.

To monitor offline data, we've provided GOFF.EXE and

GMONITOR.EXE on the disk.

COPYRIGHT/WARRANTY

GMON, GMONX, GMONITOR, and GOFF are provided to you at no charge

on the PACTERM distribution diskette. Please keep in mind,

however, that the programs are copyrighted. The contents of these

files, the software on the diskette, or the firmware in your KAM

or KAM Plus may not be reproduced in any form without the

written permission of Kantronics. However, you are granted the

right to print this documentation file for personal use. We

recognize that it is inconvenient to refer to it on screen. 'HOW

TO PRINT' instructions are included in the next section.

Regarding warranty, Kantronics does not warrant this program

since we provided it at no cost. However, we will provide limited

support for it. We suggest that you fax questions or suggestions

for the program to our service department: ATTN KANTRONICS

SERVICE DEPARTMENT, 913-842-2021. All purchased items are covered

by the Kantronics Limited Warranty Policy, a copy of which is

printed in each reference manual.

BACKGROUND ON G-TOR

G-TOR, short for Golay-TOR, is an innovation of Kantronics. G-TOR

was implemented in the KAM Plus and KAM Enhancement Board in

early 1994 and establishes a completely new hybrid-ARQ HF digital

communications system for the amateur service. Golay error

correction coding forms the basis for G-TOR. The effective

performance of two stations linked in G-TOR relies on the

combined use of forward error correction, data interleaving, and

its ARQ cycle with CRC checking.

G-TOR operates as a synchronous ARQ mode, like AMTOR and Pactor.

At 300, 200, and 100 baud, G-TOR's ARQ cycle remains at 2.4

seconds. All data frames are 1.92 seconds in duration and contain

one control byte and two checksum bytes (AX.25 CRC). Data frames

contain 69, 45, and 21 data bytes at 300, 200, and 100 baud

respectively. Acknowledgements are 0.160 seconds in duration.

G-TOR activity is found on all of the HF bands, but particularly

on 20-meters. As with AMTOR and Pactor, G-TOR uses frequency

shift keyed signals (FSK or AFSK), and you tune a G-TOR signal as

you would AMTOR or Pactor. Look for G-TOR activity in the

frequency range 14.070 to 14.085 Mhz.

DOCUMENTATION FILES ON THE DISKETTE

The documentation for GMON is stored on your PACTERM distribution

diskette under the file name GMON.DOC. To read the file, type the

DOS MORE command at your keyboard:

a:MORE <GMON.DOC

and hit return.

Your computer will display one screen of information at a time.

Press any key to go to the next page.

To print the file, type the DOS TYPE command at your keyboard:

a:TYPE GMON.DOC >PRN

and hit return.

For more information on the MORE and TYPE DOS commands, consult

your DOS manual.

PROGRAM OVERVIEW

Throughout this documentation, any reference to the operation of

GMON will also apply to GMONX (for 8088 based PCs).

GMON monitors G-TOR activity by working in concert with your KAM

or KAM Plus and your HF receiver. Your receiver should be set to

lower-sideband (SSB) to properly receive G-TOR mark and space

tones. Also, the KAM or KAM Plus must be placed in SCAN mode

using the GSCAN command. The KAM or KAM Plus delivers a stream

of data to your computer at its serial port. GMON processes

these data and displays any 300, 200, or 100 baud G-TOR frames,

including connect or disconnect frames, that are received

correctly. Both data and parity frames are displayed.

GMON also includes a trace feature; that is, you may display, in

hexadecimal format, the raw data each frame received correctly.

However, when errors are detected, GMON does not attempt to

reconstruct a frame from a data frame 'and' a parity frame

because your listening station is not a part of the link.

Hence, GMON does not, in general, receive sufficient information

from both stations to reconstruct data from mutilated frames. As

long as you are able to receive the two stations in a G-TOR link

reasonably well, you'll be able to monitor the QSOs well.

INSTALLING GMON.EXE

Copy GMON.EXE, GMONX.EXE, GOFF.EXE, and the GMON.CFG files onto

your hard drive (or run from a floppy if necessary). You may

simply copy all files from the distribution disk if you prefer.

Next, you must be sure the GMON.CFG file is properly configured

for your system. The default configuration file is set up for

9600 baud serial port operation on COM1 of your computer. If you

use a different ABAUD rate in your KAM or use a different serial

port, refer to the appendix for configuration information. To

start GMON, simply type GMON at the command prompt of the

directory where you stored GMON and press return. The GMON

screen will then appear with the help window and Kantronics'

Copyright notice displayed. Press any key to remove the help

window.

*****************************************************************

*                                          .help window appears *

*                                          .here when you hit F1*

*        --display window--                .................... *

*                                                               *

*    several more lines...                                      *

*                                                               *

*****************************************************************

com1:9600,N,8,1                             MONTRCINVBELDCDCTSRTS

(cursor, type-ahead buffer)______________________________

Characters you type from the keyboard will appear in the type-

ahead buffer line at the bottom of the screen. Characters from

the KAM or KAM Plus will appear in the display window. The HELP

WINDOW will appear at the upper right after pressing F1. Data

received from the TNC will clear off the help window.

HOW TO OPERATE GMON

Let's start with the help window. Once the program is loaded,

you can display the HELP WINDOW by pressing function key F1.

This window displays the available function and alt keys.

HELP WINDOW

GMON:  MULTIPORT TERMINAL PROGRAM

VERSION 1.01

COPYRIGHT 1994 KANTRONICS INC. CO.

ALL RIGHTS RESERVED.

F1     Display Help Menu

F2     Change Window

F6     Toggle RTS

ALT-A  Abort Transmit File

ALT-B  Toggle Log Buffer Enable

ALT-C  Clear Log Buffer

ALT-D  Execute DOS Command

ALT-F  Load File to Transmit

ALT-I  Invert GTOR monitor data

ALT-L  Save Log Buffer to File

ALT-M  Monitor GTOR

ALT-T  Trace GTOR

ALT-Q  Toggle Quiet (Bell sound)

ALT-W  Wipe Receive Window Clear

ALT-X  Exit: return to DOS

Press any key to return

When you start GMON, it runs simply as a terminal program. Items

you type at the type-ahead buffer are sent to the TNC. Items

received from the TNC are displayed on your screen. Once you

have started GMON, you should be able to press the return key

and see a command prompt (cmd:) from your KAM Plus or KAM with

Enhancement Board. If you do not see the command prompt, you may

have your GMON.CFG file set up improperly, or your normal

communications program may not have returned the KAM to its

command mode. Some terminal programs (such as KA-Gold or XP-KAM)

will leave your KAM in HOST mode. If this occurs, you must first

exit the host mode before you will be able to monitor G-TOR.

To exit the host mode, press and hold the ALT key on your

keyboard and type 192 on the numeric keypad, then release the

ALT key. Next, type the letter q. Then repeat the ALT-192

sequence and your KAM will exit the host mode and display a

Kantronics sign-on message.

TO CAPTURE A QSO OR INFORMATION coming from your TNC, hold the

ALT key down and press B; that is, press the ALT-B combination.

You will note that the colors of the characters received on the

screen will change while capturing data. This denotes that the

information is being stored to GMON0.LOG, the default log file.

If you press ALT-B again, logging will cease. At this point, we

recommend that you rename the file just logged by pressing

ALT-L. This will rename GMON0.log to whatever you specify.

Additional logging, initiated by the ALT-B directive, will again

be logged into the file GMON0.LOG.

TO TRANSMIT A FILE, press ALT-F, type the file name, and hit

return. If you wish to transmit a file from a directory other

than your program directory, you'll need to specify the path.

GMON uses only hardware flow control, so make sure you've used a

5-wire cable with your KAM or KAM Plus so that no data will be

lost.

TO MONITOR A LINK between G-TOR stations, you must configure your

KAM Plus or KAM with Enhancement Board for HF operation only and

you must set it in SCAN mode. Configure your unit for HF only

operation by setting MAXUSERS to 1/0. Enter scan mode using the

GSCAN 1200 command. For example, type the following at the command

prompt:

cmd:MAXUSERS 1/0

cmd:GSCAN 1200

GMON requires that you enter exactly 1200 as the scan rate, so

make sure you follow GSCAN with 1200. If no error messages appear

at this point, then hit ALT-M to start monitoring.

ALT-M          to start monitoring.

hit again to stop monitoring.

Tune in the G-TOR broadcast on your receiver as you would an

AMTOR or Pactor signal. Make sure that both ends of the KAM or

KAM Plus bargraph are lit. Slightly adjust tuning if you don't

see any data displayed on your screen. Portions of lines, or full

lines, of data should appear every 2.4 seconds if received

correctly.

You can recognize G-TOR signals by their duration. Each data

frame is nearly 2 seconds long, and sometimes you'll be able to

hear the acknowledgement coming back from the linked station (160

ms long). G-TOR data frames are roughly twice as long as Pactor

frames and more than four times the duration of AMTOR frames.

You can display G-TOR data frames in their raw form by pressing

ALT-T (after ALT-M has been evoked). The frames will then appear

as a string of HEXADECIMAL numbers. 100, 200, and 300 baud frames

are displayed as 1, 2 or 3 lines of HEX respectively.

You can stop trace at any time by typing ALT-T again. You can

quit monitoring G-TOR by typing ALT-M. The status of these

switches (toggles) are denoted at the bottom right of the GMON

screen. If your have a color screen, MON will be red when on.

TO QUIT SCANNING, type Control-C. This action will terminate the

GSCAN mode and return the KAM to command mode. If you desire to

continue with VHF activity at this time, remember to restore your

unit for VHF operation using the MAXUSERS command. For example,

at the cmd: prompt, enter MAXUSERS 1/1.

WARNING: If you don't return the KAM to command mode (from

scanning), it will be in scan mode the next time you use it.

TO QUIT THE GMON PROGRAM, type ALT-X. The program will take you

back to DOS.

GOFF, ANOTHER PROGRAM, DISPLAYING OFF-LINE

If your PC can't keep up with the processing necessary to display

G-TOR frames in real-time using GMON, you may wish to try storing

the raw frame data and displaying it later with GOFF.EXE.

To store raw G-TOR data using the GMON program, initiate scanning

without monitoring. We recommend that you set the BELL feature

OFF when capturing raw G-TOR data to prevent your computer bell

from ringing repeatedly. To do this, type ALT-Q. The BEL flag

will change colors to indicate the bell is disabled. After you

have the bell disabled, type the following at the cmd: prompt:

cmd:GSCAN 1200

You'll note that we did not toggle monitoring on, i.e. ALT-M.

Once a G-TOR signal is tuned in, using the KAM or KAM Plus

bargraph display, hit ALT-B to start file logging of the raw

data. The raw scan data will be stored in a file named

GMON0.LOG. Stop ALT-B when you've had enough. Keep in mind that

LARGE FILES WILL BE GENERATED at the rate of 150 characters per

second (9,000 char/min or 540,000 char/hr)! Don't start if you

don't have sufficient disk space.

TO TERMINATE SCANNING, stop logging (if on, type ALT-B) and turn

off scanning by typing control-C. Once you've stopped scanning,

exit the GMON program by hitting ALT-X.

TO ENTER THE GOFF PROGRAM, type as shown below:

a:GOFF GMON0.LOG

and hit return, or

a:GOFF GMON0.LOG +t

and hit return if you want to log traces as well as data.

Let the program run at this point until you have observed all you

wish too. Then you can abort display off line.

Terminate GOFF by typing control-C, or GOFF will terminate when

it has processed all of the data in the file.

You may alternately use the GMONITOR.EXE program to examine the

offline data. GMONITOR is a more sophisticated off-line

monitoring program, but may require considerably longer time to

process the data. GMONITOR can attempt to display the data

whether it was received "normal" (LSB) or "inverted" (USB). It

also has the ability to attempt Golay error correction on the

received data, and thus may provide more thorough data recovery.

If the captured data was from a station with a strong signal,

GOFF will probably provide you will all of the data.

To start GMONITOR, type GMONITOR and the name of the file,

followed by any options. For example:

GMONITOR GMON0.LOG +c

The +c tell GMONITOR to attempt to recover frames using Golay

error correction. To display other options, simply type GMONITOR

without a filename and a short help display will appear.

APPENDIX

CONFIGURING GMON

We have provided several sample configuration files on your GMON

disk. The default file (GMON.CFG) is configured for COM1 and an

ABAUD setting of 9600. Within the GMON configuration files, any

line beginning with a semi-colon (;) is a comment. We have

commented the configuration files so you may review them and

change those items you wish.

You may configure GMON to have a maximum of two windows (and

therefore 2 COM ports). In addition, you can edit the

configuration file to change the colors of the various windows

(foreground and background), serial port speed, and the size of

your keyboard window.

GMON supports the use of any COM Port by specifying the correct

ADDRESS and IRQ for that port. Only IRQ lines 2-7 are supported.

COM1 uses ADDRESS 0x3F8 and IRQ 4, and COM2 uses ADDRESS 0x2F8

and IRQ 3.

DO NOT USE ANY OTHER ADDRESS OR INTERRUPT UNLESS YOU ARE SURE OF

THE PROPER SETTINGS.

IN CASE OF TROUBLE

In case the program or the KAM locks up, terminate program

operation and turn the KAM off and back on. Then start over.

If you have difficulty with the KAM upon returning to modes other

than GSCAN, consider the following: 1) Perhaps the KAM was left

in GSCAN mode? 2) Perhaps MAXUSERS was not restored from 1/0 to

its original setting? 3) Perhaps ALT-B was left on from logging

and your disk filled up?