*************************************************************************

PC-PAKRATT II

RELEASE NOTES

************ Jan. 29, 1991 ******************** ver. 4.0D ***************

UPGRADING FROM PC-PAKRATT

There is NO backward compatibility to the original version

of PC-Pakratt.  Any macros and message files used with PC-

Pakratt will have to be recreated for PC-Pakratt II.  You

may wish to create a new sub-directory for PC-Pakratt II so

as to keep your PC-Pakratt files for reference.  Since PC-

Pakratt II is started with the command pp, any batch files

that you may have created for PC-Pakratt will have to be

changed for PC-Pakratt II.

When PC-Pakratt II is starting up, the program will read the

release date of the firmware in the PK-232 and display the

results in the upper right corner of the initialization

screen.

OPERATING SCREEN, ANY MODE

The percentage of the scroll back buffer that has been used

is now displayed on the screen when in any mode.  To reset

the counter to zero, the scroll back buffer will have to be

cleared with a CTRL-PAGEUP.

MOUSE NOTES

If you have a Logitech Bus Mouse, and you find that the

cursor disappears about half way down the screen, then you

may need a newer version of MOUSE.COM.  We have found

problems with MOUSE.COM files dated in September 1987.

Logitech is a trademark of LOGITECH Inc.

UNKNOWN COMMAND ERRORS

To disable "Unknown Command" errors, see page 25 in the

PC-Pakratt II Operating Manual or the -[MODE] SWITCH

section below.

-[MODE] SWITCH, PAGE 6

If you want to have PC-Pakratt II boot straight into a

communications mode, (for example, typing pp -packet will

cause PC-Pakratt II to boot straight into the packet mode),

then you may want PC-Pakratt II to ignore any unknown

commands.  PC-Pakratt II can be configured to ignore any

unknown commands by setting the "Unknown Command Errors"

option to be disabled.  Start from the opening menu and

select "PC-Pakratt Parameters".  From there set "Unknown

Command Errors" to be disabled.  Be sure to save this change

in PC-Pakratt II with the "Save PC-Pakratt Configuration"

option in the "Files Menu".  See page 25, "Turning Off

Unknown Command Errors" in the PC-Pakratt II manual.

HOT KEYS, PAGE 11

The hot keys will not be visible if the -bw switch is used

to start PC-Pakratt II.  Use the cursor up / down keys to

make your selection.

CONFIGURATION MENU, PAGE 12

The option to "Set COLOURS" has been removed.  The changing

of colors is now done from the "Communications Menu" from

the selection "Change Colours".

AMTOR/BAUDOT/ASCII/MORSE SETUP SCREEN, PAGE 16

The command XMIT OK is an ALL MODE command and has been

added to this setup screen

FILES MENU, PAGE 18

The nomenclature for the first two items in the "Files Menu"

have been changed.  "Save Configuration" has been changed to

now read "Save PcPakratt Configuration" and "Save Parameter

Defaults" has been changed to now read "Save PK-232

Parameters".

PCPAKRATT PARAMETERS MENU, PAGE 25

The option "Rx Window (43 line mode)" now reads "Rx Window

(High Res mode).   When the High Res Mode is enabled, (high

res is enabled with the -h command line), PC-Pakratt II will

go to the 43 line mode if an EGA video card is detected and

to the 50 line mode with a VGA card.

If VGA is detected, the maximum number of lines in the Rx

window can be 44 otherwise an EGA is assumed and the maximum

of lines will be 37.

SAVING SCROLL BACK BUFFER TO DISK, PAGE 33

A new entry is available when in the scroll back screen.

The F2 key will now save the entire contents of the scroll

back buffer to disk.  If the scroll back buffer should fill,

the "top" 25% will be cleared to make room for more

incoming data.

F10 KEY IN PACKET, PAGE 36

The operation of the F10 key in packet has been enhanced

so that when switching between VHF and HF, the HBAUD

parameter will now also change.  The HBAUD setting for VHF

will be 1200.  The HBAUD setting for HF will be 300.  The

F9 key can change the value of HBAUD independently.

TRANSMIT ECHO IN PACKET

An additional option has been added to the PcPakratt

Parameters Menu called "Packet Echo".  When this feature is

enabled, transmitted packets will be echoed to the receive

buffer part of the screen.  Packet Echo is disabled and

enabled in the PcPakratt Parameters menu.  Remember to save

any changes you make before leaving PC-Pakratt II.

FAST INITIALIZATION

An additional option has been added to the PcPakratt

Parameters Menu called "Fast Initialization".  When enabled,

PK-232 parameters will NOT be uploaded from the program disk

to the PK-232 when starting PC-Pakratt II.  This will

significantly reduce the amount of time required to boot PC-

Pakratt II.  The default setting has Fast Initialization

disabled.

When starting PC-Pakratt II, the status of Fast

Initialization will be assessed, and if enabled, PC-Pakratt

II will then determine if the PK-232 is still in the host

mode.  If the PK-232 is found to be in the host mode, then

the PK-232 parameters that are stored on disk will NOT be

uploaded to the PK-232.

Two additional command line switches have been added that

affect Fast Initialization.  Starting PC-Pakratt II with a

pp -f will force PC-Pakratt II to initialize in the "Fast"

mode regardless of the setting of Fast Initialization.

Starting the program with a pp -i will force PC-Pakratt II

to upload from disk, all PK-232 parameters.

If PC-Pakratt II will be restoring PakMail messages from

disk, Fast Initialization will be disabled.  See

"SAVE/RESTORE PAKMAIL", next.

SAVE/RESTORE PAKMAIL, PAGES 27 and 40

The discussion about "Save/Restore Mail" does not take into

account the July 1990 firmware release.  Starting with the

July 1990 firmware release, the internal battery in the PK-

232 CAN BE LEFT INSTALLED.  The internal battery will now

back up PakMail messages in the PK-232.

If "Save/Restore Mail" is enabled and the PakMail mailbox is

empty, the file PAKMAIL.MES will not be created.

The restore PakMail messages to disk function has been

changed making the second to the last sentence on page 40

incorrect.  If "Save/Restore Mail" is enabled, message

restore from disk will occur only if the mailbox has been

found empty during program startup.  Restoring PakMail

messages can be still invoked from the PakMail menu in the

Packet Communications Screen (F2 Key).

BATCH FILE EXAMPLE, PAGE 41

The last line in the batch file example is in error.  The

line "CD\PC-Pakratt II" would not work.  DOS only allows up

to eight characters and no spaces.  The recommended

directory for PC-Pakratt II is PCRATT2.  Therefore, the last

line of the example batch file should read "CD\PCRATT2".

If you run the CORRECTED batch file, (see "Batch File

Example, Page 48" below), then PK-FAX and it's assorted

files DO NOT need to reside in the same directory as PC-

Pakratt II.

INPUT EDITING

When an input is required by PC-Pakratt II, the previous

value for the input will be displayed.  This is done so

that if a minor change is needed (e.g. to add a -1 to the

end of a call sign), one can simply use the cursor keys to

move to the point of change and type it in (without re-

typing the entire value).  If the new value is completely

different than the old value and is shorter as well, the

"left-over" characters do not need to be deleted.  Example:

replacing WA3ABC with N7AA will leave N7AABC on the screen,

but only the new value will be accepted, and the additional

letters "BC" do not need to be deleted.

YAPP BINARY FILE TRANSFER

When using YAPP as your binary file transfer program, two

changes will have to be made to the initialization string in

YAPP.  The following is a sample of the last few lines from

a modified initialization string.

CTRL-A OHOOFF CTRL-W

mon on

conok on

*** EOF  {end of commands sent to TNC on program start}

conok off

mon off

host on

*** EOF  {end of commands sent to TNC on program end}

Before YAPP starts sending commands to the PK-232, the host

mode will have to be turned off.  In the above example, the

first line performs this function.  When leaving YAPP, the

last command sent to the PK-232 must be the host on command

as seen above.  Permission to print the YAPP initialization

screen courtesy of Jeff Jacobson.

BATCH FILE EXAMPLE, PAGE 48

The second and third line of the example batch file are in

error.  The second line should read PKFAX.  The line

"CD\PC-Pakratt II" would not work.  DOS only allows up to

eight characters and no spaces.  The recommended directory

for PC-Pakratt II is PCRATT2.  Therefore, the last line of the

example batch file should read "CD\PCRATT2".

ADDED COMMAND LINE SWITCHES

Additional command line switches have been added.  They

are -bw and -color, with -color being the default.  If you

are working with a computer which uses an LCD or Monochrome

screen, then -bw can be added to the command line when

starting PC-Pakratt II.

For use with the Fast Initialization feature, two command

line switches have been added.  The -i will force parameter

upload and -f will prevent it.  See "FAST INITIALIZATION"

above.

CLEARING THE TRANSMIT BUFFER

Pressing an ALT-T to clear the on screen transmit buffer

will now clear the any data in the PK-232 transmit buffer.

RETURN TO RECEIVE MODE

When in the RTTY type modes and in CW, inserting a CTRL-D

at the end of transmitted text will force the PK-232 to

return to the receive mode when the CTRL-D is

encountered.  To embed a CTRL-D in the transmit buffer,

simply press and hold the CTRL key followed by the letter

D.  This same procedure works in both the transmit buffer

and when writing macros.

CLEARING MFROM, MTO, DFROM AND CFROM CALLSIGNS

To remove a callsign from one of these parameter lists, move

the courser to the desired callsign, press the spacebar then

press the RETURN key.

DOS VERSION UPDATE

PC-Pakratt II with Fax has been tested and found to be

compatible with both IBM DOS 4.01 and MS-DOS 4.01.

IBM is a Registered Trademark of International Business

Machines Corporation.  MS-DOS and Microsoft are Registered

Trademarks of Microsoft Corporation.

The auto-save start and stop times are not saved in the

configuration file.

PS/2 MODEL 50Z USERS

If your IBM PS/2 Model 50Z was manufactured prior to

September 1988, your computer may be in need of a new System

Control Board.  Contact your IBM Dealer or AEA.  PS/2 is a

trademark of International Business Machines Corporation.

LEADING EDGE MODEL D USERS

Some customers have reported problems with PC-Pakratt II when

used on machines with a BIOS version lower than 3.00.  A

Leading Edge Model D may need a BIOS version of 3.00 or

later to work with PC-Pakratt II.

If the serial port on your computer uses a DB-9 type of pin

configuration, a DB-25 - DB-9 adapter will be required.  A

DB-25 to DB-9 adapter that is available from your computer

dealer should be acceptable.  Be sure to note the specific

gender that is required for your serial board.

PRINTING FILES WITH PK-FAX

When printing the contents of the buffer, PK-FAX will print

to the printer that is attached to the computer as described

in the PK-FAX configuration screen.  PK-FAX will not print

to the parallel port on the "Y" cable you received with the

PK-232.

PC-PAINTBRUSH FILES

Any picture created with PC-Paintbrush will have to be

created with PC-Paintbrush configured in the "two-color"

mode.  Run your PC-Paintbrush install program and select the

appropriate two-color mode for the video display used on your

computer.  You must reset the computer before the new

selections become active.  PC-Paintbrush is a trademark of

ZSoft Corporation.

MANUALS

When a conflict occurs between the PK-232 Users Manual and

either the PC-Pakratt II or PK-FAX manual, use the

information in the software manual!

RS-232 CABLES

PC-Pakratt II requires pins 2, 3, and 7.  PK-FAX requires pins

2, 3, 5, and 7.  If you build an RS-232 cable, be sure to

use pins 2, 3, 5, and 7.  The cable supplied with the PK-232

works with both PC-Pakratt II and PK-FAX.

UPDATE INFORMATION

Update announcements of PC-Pakratt II and the PK-232, are

mailed to our users via the information that we receive from

the warranty cards.  Please return your warranty cards.

WINDOWS 3.0 APPLICATION NOTES

PC-Pakratt II has been found to behave itself when run under

Windows 3.0 under the following conditions.

o  Must run the 386 Enhanced Mode.  We have had no

experience running PC-Pakratt II in the "Real" or

"Standard" mode.

o  PC-Pakratt II can run as a window.  When creating

your PIF file, make sure that the box labled

"Background" under "Execution" has been checked

and that there is an "X" in the box.

Windows is a trademark of Microsoft and 386 is a

trademark of Intel Corporation.

PK-FAX ENHANCEMENTS

Added support for the COM3 and COM4 serial ports.  The

options lists for the "Printer Configuration/Printer port"

menu and the "PK-232 Configuration/PK-232 serial port" menu

now include COM3 and COM4.  The user may use the "COM3/COM4

configuration" menu under "PK-232 Configuration" to alter

COM3 or COM4 parameters such as IRQ bit, vector and base

register.

The printer control codes may now be customized for printers we

haven't covered specifically in PK-FAX.  Here is how to do

it:

1. Under "Printer Configuration," set "A. Printer type" to

the type of printer whose control codes come closest to the

printer being set up.  Then set "A. Printer type" to Custom.

2. Select "B. Printer graphics density" and choose the

desired dot density.  The density is expressed in dots per

inch, which assumes a printing width of 8 inches.  Some

printers have a usable width narrower than 8 inches.  The

user should divide the desired number of dots by 8 and

select that number.

3. Select "I. Printer control codes" and change the

appropriate codes.  The codes must be entered in

hexadecimal, not decimal or ASCII characters.  The changed

codes stay in effect as long as "Printer type" is set to

Custom.

The Printer Control Codes menu contains the following items:

A. Set graphics mode               Sent each line before the

image data.

B. Set line spacing                Sent once at the start of

the image.

C. Do line feed                    Sent each line after the

image data.

D. Reset printer                   Sent once at the end of

the image.

E. Vertical dots                   Shows number of vertical

dots and their

orientation.  Set the

number with the "Printer

data bits" command under

the "Printer

Configuration" menu.

F. Horizontal dots                 Shows number of

horizontal dots.  Set

the number with the

"Printer graphics

density" command under

the "Printer

Configuration" menu.

Example:  The Tandy DMP-132 is not covered in PK-FAX.  Set

"Printer type" to Radio Shack and then Custom.  Then set

"Printer graphics density" to 60 dpi to yield 480 dots

across the paper.  Select "Printer control codes" and set

the following:

A. Set graphics mode      12

C. Do line feed           0A

D. Reset printer          1E 0D 0A

It's important to set "Printer type" to the correct printer

before going to Custom.  Some printer types are handled as

special cases in PK-FAX:

HP LaserJet & ThinkJet receive dot data in a horizontal

format rather than vertically as most other printers do.  In

higher densities horizontal lines need to be received twice.

Radio Shack and Gorilla type printers need to have the

unused bit 7 of the data byte set to 1.

Okidata printers need to receive twice any data bytes

that happen to be equal to 3.

Toshiba printers must have the 8 data bits expanded out to

24 bits.

TECHNICAL SUPPORT

To obtain support for this or any other AEA product:

IN THE U.S. AND CANADA:

AEA, Inc.

P.O. Box C2160

Lynnwood, WA  98036-0918

tel: (206) 775-7373

fax: (206) 775-2340

IN EUROPE:

ICS Electronics

Unit V, Rudford Industrial Estate

Ford, Arundel, West Sussex

England  BN180BD

tel: 0903 731101

NOTE:  To get a hard copy of these notes on your printer,

print the README.DOC file and not this batch file.

##########################################################################