FAI COLUMN

Editor:Dom DEHAYS F6DRO

Please send your FAI reports to f6dro@aol.com

FAI for beginers

I have been recently asking for FAI reports ( Field aligned irregularities) on several VHF reflectors, and I was very surprised to read that many guys have not the slightest idea of what FAI is. So I decided to write a simple artical about it.

What is FAI ?:

It is a particular VHF propagation mode in wich a volume of space ( wich is believed to be aligned on a magnetic field irregularity) acts like a scatterer for electromagnetic waves encountering it.

Wich frequency ? :

In our ham bands , 6m and 2m are frequencies of choice for FAI QSOs ..I recall having heard about a 70cm QSO but never saw any details about it , so it is probably myth. On 6m , because of the relatively broad antennas we are using , it is not easy to recognize we are using FAI rather than sporadic E during a contact , well trained operators identify FAI by the flutter associated with it , sometimes auroral CW is heard : it is a FAI symptom too.

On 2m , it is easier to identify , because maximum signal strength is not obtained by turning your antenna to your QSO partner azimuth , but rather to a place located northernly ( in the northern hemisphere) of the great circle path. ( it is the scatterer location). Flutter is also there as well as the auroral tone on CW ( but this auroral tone is not always there).Signals are weak except for big guns… as usual. Depending how far you are located from the scatterer , elevating your antenna increases signal strength .

When do we encounter FAI ? :

Fai conditions are associated with sporadic E. So we encounter it mostly during the sporadic E season from may to august. But Es can occur at any time of the year and I have often heard stations outside the traditionnal season , simply because their antenna was accidentally beamed to a scatterer. Don’t forget there is a second pronounced peak in Es activity at the beginning of january , and very often I can hear guys via FAI during the Quadrantids shower , simply because there is Es around and activity at the same time.

Wich mode :

Fai is a weak signal mode , so CW or SSB are alright.

Wich equipment :

Let’s say we use 144Mhz , because it is there FAI is more productive . Of course the bigger the better , but a good starting point is 100w , good RX and a 3 lambdas yagi , if possible elevation is much better as you can get a 10db increase in signal strength depending how far the scatterer is located from you. I managed to work many qsos using only 10W in a single 13el F9FT with fixed 10 degrees elevation , so QRP is possible.

Wich distance ?

Depends of your location see DF5AI article (Dubus 3/2002).

More details about the special geometry of this FAI scatterer :

First of all : why should the scatterer located northernly of the great circle path ?

This is because of the orientation of magnetic field lines. ( north of the great circle path in the northern hemisphere)

A special geometry between QSO partners is needed for a qso to be possible :

This has already been discussed in details by DF5AI in DUBUS 3/2002 p32 . I will not repeat it there.

What must be understood here is that for a given scatterer location you have only a limited coverage , that is to say not all stations seeing the scatterer can work together , you need to satisfy a given geometry. This possible path is given by drawing isoflectionnal curves centered on the scatterer.

Isoflectionnal curves exemple :

QSO on 5 august 2002 , between F6DRO ( JN03tj) and 9A1CAL ( JN86el) at 1240z.

Scatterer location found from both stations given azimuth for the qso to take place.

SCP

90°

So called isoflectionnal curves have been drawn , one line has a special status labelled “ 90 ” here. For a QSO to be possible you must find a partner located on the same isoflectionnal line than you , but with the opposite sign from “ 0 ” line . F6DRO in JN03 in located on the isoflectionnal line wich is  4 lines south from the zero line , and 9A1CAL is located four lines north from the “ 90 ” line. F6DRO can work all stations located on the same line than 9A1CAL .

FAI : in wich place ?

It was believed that fai was only possible in southern europe , simply because many reports came from guys , using a very efficient scatterer , located above JN36 . Another very active scatterer was found over Budapest , but since , other scatterers , in JO20 and JO63 were used by stations outside the traditionnal area where FAI was reported at the beginning. In fact , any sporadic E « cloud » , may give FAI contacts , but you must be lucky to find a partner at the right location .

Software tools :

Several software can give you the oportunity to analyse , and predict FAI .

For MacIntosh platform , the very good software written by DF5AI callesd BEAMFINDER , you can find more details on it on : www.df5ai.net

For DOS/WINDOWS platforms use K9SE FLD-ALGN , and SE-PROP for Es analysis : www.home.netcom/~wb9qiu

FAI more in depth :

We will see in second part of this article , a few detailed cases of FAI propagation , and a suggested strategy to use this propagation mode