TI Downloader Program

The Downloader program must be installed using the setup.exe program.

Follow the prompts and complete the installation. If the installation doesn’t complete, run the setup.exe program a second time.

The Downloader program can be programmed with command line arguments. This can be done by setting up a shortcut to the program and then setting the properties. The download.exe program is placed in the Windows directory so you will not have to include the path to download.exe.

download.exe [/Ffilename] [/Xfrequency] [/Pport] [/Bbaud(download)] [/H] [/T[baud]] [/S] [/D]

All operands should immediately follow the switch character with no spaces. i.e.:

download.exe /Fconv.hex /B9600 /P1

If the filename, crystal frequency or port are not included then a screen will prompt for the values.

/Ffile		hex file, #H in the Keil environment will substitute the hex file (required).

/Xfeq		MSC1210 Clock frequency (required)

/Pport		PC Comm port 1, 2, 3 or 4 (required)

/Bbaud		Baud rate computed from xtal otherwise, standard rates

/H		If this flag is present the configuration bytes will be erased

/Tbaud		Requests a terminal window at 0ptional (baud) rate

/S		Skip downloading and go directly to the terminal

/D		Debug, Provides a small terminal of commands during download

/Esize	Only erase (size K bytes) portion of program memory,  This has no effect on Data Flash in the Y2, Y3, and Y4 devices since the Data Flash is not accessible during programming on those devices

/M	Manual Reset (wait for confirmation before download)

If the /Xxx or /Pn command line parameters are not supplied, the Downloader program will wait for the arguments with the following screen and open a dialog box for the file name. The Baud rates for Terminal and Download are only shown if they are selected.

After these arguments have been supplied the program will attempt to download the Intel Hex file to the MSC1210EVM. The serial communications with the MSC1210EVM board uses the connector for SIO0. Using the information provided the Downloader program selects a baud rate, initializes the registers in the MSC1210 and then transfers each line of the Hex file to the MSC1210 while showing the progress with the following display.

When the downloading is finished, if the /T parameter is specified (or show terminal selected), the program opens a terminal program which can be used to communicate with the MSC1210.

The buttons have the following functions

Open Log File	Provides a method to log the  activity to a file

Close Log File	End logging and close the file

Properties	Change Comm Port or properties

Load Basic File	Used to retrieve a File and send it to the Comm port. Starts with NEW command

Save Basic Prog	Issues LIST command and copies results to a file.

Reload Hex File	Sends again the Hex file previously sent to the MSC1210

Reset		Resets the MSC1210EVM by toggling the DTR signal.

Power Cycle	Turns off the power for the MSC1210-DAQ-EVM which is powered from RS232 signals

Clear Screen	Clears the terminal display

Under the File menu is listed the last command line arguments supply. Any argument can be selected, edited and then used to restart the Downloader program with the new parameters.

The uVision Keil environment provides a method to call the download.exe program when it finished a successful compilation. Simply select “Run User Program #1” and fill in the parameters desired.

It is also useful to define a command in the uVision Tools menu to perform the download operation. Then the download can be invoked without first doing a compilation. Just create a command name ‘download’, then enter the parameters for that command in the fields specified.

The /H flag provides a means to program the Hardware Configuration Registers. They are located in a separate 128 bytes of Flash memory. During serial programming these bytes are mapped from address 0x8000 to 0x807F. The /H flag directs that these bytes be erased so that they can then be programmed with new values.

By adding an assembly language file to the project, these can be specified as follows:

; Define HCR0 and HCR1 while download Flash Programming

CSEG AT 0807EH

DB	0FCH	; 76:DBLSEL 54:ABLSEL 3:DAB 2:DDB 1:EGP0 0:EGP23

DB	0FFH	; 7:EPMA 6:PML 5:RSL 4:EBR 3:EWDR 210:DFSEL

END

While downloading the file this screen is displayed. The Total Lines are the number of lines in the Hex file. The baud rate used is the baud rate specified in the /B parameter or the baud rate calculated from the /X parameter. If a terminal is specified, then it will default to the same baud rate that is used to download the program unless the /T is followed by a number to represent the desired baud rate.