CONTENTS:

- GENERAL DESCRIPTION

1.1 	TECHNICAL DATA

1.2 	PRINCIPLE OF OPERATION

2. 	SERVICE

2.1 	MAINTENANCE

2.2 	RECOMMENDED MEASUREMENT INSTRUMENTS

2.3 	PERFORMMJCE CHECK

2.4 	SELF MADE TEST LOAD

2.5 	NECESSARY ADJUSTMENTS AFTER REPAIR

2.6 	TROUBLE SHOOTING

2.7 	PIN CONFIGURATION

3. 	CIRCUIT DESCRIPTION AND DIAGRAMS WITH MEASUREMENTS

3.1 	INPUT FILTER (MODULE 400)

3.2 	POWER UNIT I

3.3 	POWER UNIT II

3.4 	BLOWER CONVERTER (MODULE 300)

4. 	COMPONENT LOCATIONS:

4.1 	CAPACITOR UNIT (MODULE 100)

4.2 	PU I AND PU II CONTROL UNIT (MODULE 200)

4.3 	BLOWER CONVERTER (MODULE 300)

4.4 	INPUT FILTER AND FUSES (MODULE 400)

4.5 	CHASSIS MOUNTING (MODULE 500)

4.9 	DISASSEMBLING FROM Tl130

4.10 	MAIN DIAGRAM

5 	PART LISTS

CONTENTS:

1. 	GENERAL DESCRIPTION

1.1 	TECHNICAL DATA

  - PRINCIPLE OF OPERATION

- GENERAL DESCRIPTION

SAILOR N1407 is a De power supply intended to supply a SAILOR SSB short wave station from program 1000/B, when the set has to be supplied from a 24V battery.

SAILOR N1407 is an integral part of the transmitter T1130 and all the controls of the power supply takes place via electrical wires.

SAILOR N1407 The information to control N1407 comes from the short wave station and is established from the commands given to the set via the push buttons in the rack and from the mode selected on the exciter and receiver by the operator.

SAILOR N1407 with MAIN SWITCH in position RX-ONLY. Only the receiver is in operation and low power consumption is achieved.

SAILOR N1407 is provided with a thermostat which starts the fans for the power supply, if the temperature inside the power supply gets too high

The power supply N1407 delivers all necessary voltages to a SAILOR 1000/B SSB short wave set with an output power of 400 W PEP in the frequency range 1.6 - 27.5 MHz.

  - TECHNICAL DATA

OUTPUT VOLTAGES:

DC stabilized:

OPERATION TEMPERATURE RANGE:	-150C to +550C

COOLING:

As long as the power supply supplies a receiver and an not keyed transmitter, the cooling takes place via convection only.

When the transmitter is keyed or the internal temperature of the power supply is above +550C the blowers inside the T1130 unit are continuously running and cooling down the power supply and the transmitter.

When the temperature falls below + 45ºC the blowers stop.

If temperature in transmitter is too high, the power supply is reduced from 38V to 30V. If temperature is further increasing, the Power Unit II is blocked and blowers start. When temperature is decreased blowers stop, and Power Unit II is unblocked again.

22V	+/- 2%	I max	3.2	A

8V	+/- 2%	I max	2.0	A

28V	+/- 2%	I max	3.4	A

38V	+/- 2%	I max	 14	A

DC unstabilized:

-45V 			Imax	 0.15A

AC unstabilized:

V blower = V im – 2 x V sat, I max 2A

frequency 60 Hz

Nominal voltage 24 V

INPUT VOLTAGE:

INPUT CURRENT:

Extreme voltage 21.6 - 31.2 V

by 26.4V DC input

2-tone	approx. 30  A

A3J	approx. 17  A

A3H	approx. 24  A

R.O.	approx.  2.5A

1.2. 	PRINCIPLE OF OPERATION

The power supply N1407 consists of three power supplies.

Power Unit I is a 400 Hz DC converter which delivers low power voltages for receiver, exciter and transmitter.

Power Unit II is also a 400 Hz push-pull converter which delivers high power to the transmitter.

The blower converter produces an AC voltage for the blowers.

PU I is working when the set is switched on.

PU II is only working when PU I is switched on and the transmitter is keyed. N1407 is switched ON/OFF by the switches in H1235 rack system.

Maximum loads for the output voltages:

38V	:	IL max =	14 A 	RL =	2.7	ohm (600 W)

PU II

28V	:	IL max =	3.4A	RL =	8.2	ohm (100 W)

22V	:	IL max =	3.2A	RL =	 6.8	ohm ( 75 W)

PU I 	 8V	:	IL max =	2.0A	RL =	4.0	ohm ( 20 W)

-45V	:	IL max =	0.15A	RL =	300	ohm ( 10 W)

It is necessary to have all outputs loaded with their max. load, when the current limiters are adjusted.

To ensure proper regulation it is necessary that the power supply has a minimum load.

PU I	: min. load for 22V output is approx. 22 ohm (no load at 8V and at -45V output)

PU II	: min. load to 38V output is approx. 50 ohm (no load at 28V output)

CONTENTS:

2. 	SERVICE

2.1 	MA I NTENANCE

2.2 	RECOMMENDED MEASUREMENT INSTRUMENTS

2.3 	PERFORMANCE CHECK

2.4 	SELF MADE TEST LOAD

2.5 	NECESSARY ADJUSTMENTS AFTER REPAIR

2.6 	TROUBLE SHOOTI NG

2.7 	PIN CONFIGURATION

When the SAILOR Short Wave Set type 1000/8 has been correctly installed, the maintenance of the power supply can dependent on the environments and working hours be reduced to a performance check at the service workshop at intervals not exceeding 5 years.

A performance check list is enclosed in the PERFORMANCE CHECK section. Also inspect cables and plugs for mechanical defects and corrosion.

Any repair of the set should be followed by a check described in the section NECESSARY ADJUSTMENTS AFTER REPAIR.

2.3. 	PERFORMANCE CHECK

2.2. 	RECOMMENDED MEASUREMENT INSTRUMENTS

2.1.	MAINTENANCE

MULTIMETER . . . . . . . . . . PHILIPS PM2505

OSCILLOSCOPE . . . . . . . . . PHILIPS PM3214

2.	SERVICE

- N1407 is mounted in Tl130 (1000/B rack).

- Connect 24V DC to plug Pl03.

- Push "RX-ONLY" (switches on PU I).

- Check 22V output voltage. If necessary then adjust to 22.0V with

R238.

- Check 8V output voltage. If necessary then adjust to 8.OV with R241.

- Check -45V output voltage to approx. -40V.

- Push "ON" button and press key.

- Check 28V and 38V output voltages. If necessary then adjust to 28V with R271 and to 38V with R259.

- Current limiters cannot be adjusted.

- While pressing the key or short-circuiting the thermal breaker TB501 check that both blowers are running.

- When the blowers are running then check the blower converter.

Measure with oscilloscope that the output frequency is 60 Hz (16.67 ms) and the pulse time is 7.5 ms.

2.4 	SELF MADE TEST LOAD.

SELF MADE TEST LOAD FOR NI407 / NI409

When PU II is switched on, it is always necessary to cool the switch transistors T504 and T505 and the output rectifier D505. It is recommended

to mount N1407 in a Tl130 chassis with a blower to cool the power supply.

- Remove the connection wire from the collectors of T504 and T505 to the transformer TR502.

- Check the output voltage of PU I.

- Connect scope ground to negative input voltage (negative pole of C503)

and scope probe to the base of T504.

- Replace F402.

- Switch on PU I and PU II and check the base driver (see fig. on diagram).

- Switch off PU I and PU II and connect scope probe to the base of T505.

- Switch on PU I and PU II and check the base driver (see fig. on diagram).

- Switch off PU I and PU II.

- Connect the transformer TR502 to the collectors of T504 and T505.

- Switch on PU I and PU II.

- Turn R243 out of current limiting.

Adjust 38V output with R259 to 38.0V. Adjust 28V output with R271 to 28.0V.

Adjust current limiter with R243 so the 38V output is 35.5V with full load at 38V output and at 28V output.

- Let the power supply work for about an hour and then check again the output voltages. Adjust if there has been some deviation.

2.5.	NECESSARY ADJUSTMENTS AFTER REPAIR

(using SELF MADE test load)

POWER UNIT I

- Connect SELF MADE test load to N1407 outputs.

- Remove fuse F402.

- Connect 24V DC to plug Pl03.

- Switch on PU I.

- Adjust 22V output with R238 to 22.0V.

- Adjust 8V output with R241 to 8.0V.

- Check -45V output (approx. -40V).

- Switch off PU I.

POWER UNIT II

BLOWER CONVERTER

- Switch on PU I and PU II. The blowers have to run.

- Connect scope ground to the negative pole of the input voltage and scope probe to IC302 pin 1 (or pin 2). Adjust with R305 so that the frequency is 60 Hz (16.67 ms).

- Connect scope probe to IC303 pin 3. Adjust with R311 so that the pulse time is 7.5 ms.

Failure in Power Unit I.

2.6.	TROUBLE SHOOTING

- Bad connections in plug P103 or J101.

- Fuse F401 is blown out.

- The converter transistors T501 and T502 or the output stage in the blower converter are short-circuited.

- PUI will not start up.

a) Check the relay RE201 and the voltage to it.

- The converter starts up, but the output voltages are wrong.

- Check the outputs severally.

- Failure in the relays RE203, RE204 and RE205 or in the transistors T205 and T206.

Failure in Power Unit II.

- Relay RE202 is not activated when handset key is pressed.

- PU I is out of function.

- Handset key is out of function.

- Supply is blocked.

- Temperature protection unit in Tl130 blocks the power supply because of overheating of the transistors in the power amplifier in T1130.

Check also the blowers and air filter.

- Bad connection of J102.

- Over- and under voltage shut-down at IC201.

- Supply starts up, but goes into "hiccup mode".

- Wrong adjustment of current limiter R243.

- Output voltage is short-circuited or overloaded.

- To low input voltage.

Failure in the Blower Converter.

- Blowers will not stop.

- Toa high temperature in the power supply.

- Tea high temperature in the transmitter.

- Bad connection in J102.

- Thermal breaker is short-circuited.

- Dirty air filter.

- Blowers will not run.

- On/off circuit consisting of IC301, T301 and T302 is out of function.

- Wrong frequency of IC301.

- Wrong pulse time of IC303.

PIN CONFIGURATION cont.

CONTENTS:

3. 	CIRCUIT DESCRIPTION AND DIAGRAMS WITH MEASUREMENTS

3.1	INPUT FILTER (M0DULE 400)

3.2 	POWER UNIT I

3.3 	POWER UNIT II

3.4 	BLOWER CONVERTER (MODULE 300)

3. 	CIRCUIT DESCRIPTIONS AND DIAGRAMS WITH MEASUREMENTS

3.1. INPUT FILTER (MODULE 400)

The filter consists of capacitors and a filter choke L401 to suppress switch noise. It fulfills the CISPR noise regulation.

The fuse F401 protects Power Unit land the blower converter. Fuse F402 protects Power Unit 11. The diodes D401 and D402 protects against reverse polarity of the input voltage to P103.

3.2. POWER UNIT I

In the rack system H1235 the terminals J101/32 and J101/33 are short circuited and relay RE201 is activated when starting up the PU I. The converter, consisting of TR201, TR501, T501 and T502, starts up and delivers voltage to the three secondary outputs.

-45V output is only rectified in D205 and is not regulated. The voltage is

-34 to -54V depending of the input voltage. -45V to the receiver is switched on/off by the transistors T205 and T206. They are controlled by relay RE203 which also switches on/off the 22V and 8V to the receiver.

22V output is regulated after the switch mode principle by IC202 and is adjusted with R238. Transistor T203 turns on and the current flows

through L501 and L502 to the load. The voltage increases at pin 4 and IC202 turns off T203. The current flows continuously in L501 and back through D210. The voltage decreases at pin 4 and IC202 turns on T203 again. The regulator is then self oscillating. R233 forms a current limiter.

8V output is a linear series regulator controlled by IC203. The output voltage is adjusted with R241. IC203 is supplied from 22V output which means that 8V is present only if 22V is present.

The relays RE204 and RE205 switch on the voltage to Tl130 and to the exciter.

3.3. POWER UNIT II

When the transmitter is keyed the relay RE202 switches on. The antenna tuner AT1500 gets 22V supply via J101 pin 7. The voltage at IC201 pin 6 and pin 7 is determined by R204, R205 and R206. These inputs form an over and under voltage protection. "Over voltage" shut down occurs at approx. 33.5V input. "Under voltage" shut down occurs at approx. 16.5V input.

Switching on PU II means that C207 can be charged by IC201 and create a soft start function at pin 15 to limit the peak current in the switch transistors and the output rectifier.

The IC201 has two outputs, pin 12 and pin 13, working as push-pull and being active low. The switch frequency is approx. 450 Hz, but the frequency of the internal saw tooth oscillator is approx. 900 Hz, determined by R207 and C206.

Regulation of the duty-cycle is done by varying the voltage at pin 4, This regulation signal te control the output current and output voltage is created in IC204,

Voltage supply for IC201 is regulated by means of R201 and D201.

The controlled square wave from the outputs of IC201 is led to the switch transistors T504 and T505 via the driver transistors T201 and T202 and the driver transformers TR202 and TR203. The diodes D206 - D209 clamp

the over voltage transients from the driver transformers. The diodes

D503 and D504 together with C501, R502, C502 and R503 protect the switch transistors against transients and unwanted oscillations. C503 reduces the ripple to the input filter.

The output voltage is rectified in D505. R504, C504 and R505 protect against unwanted oscillations and transients. The choke L503 is an energy reservoir. Together with Cl0l - Cl06 it is smoothing the output current.

The output voltage of 38V is regulated by IC204a. The reference voltage

is made by D213. Adjustment is done with R259. lf the output voltage increases, pin 1 will go low. Via IC201, IC201 pin 4 is pulled down and this reduces the duty-cycle and thus the output voltage. R266 has to be connected to ground to get 38V. For supply reduction to the transmitter, R266

is released from ground via the thermal protection unit in Tl130 and the output voltage is approx. 30V.

A current limiter is performed by IC204b. Rl0l is the current sensor and is made of constantan wire. lf the current exceeds the level determined by R243, the output pin 7 goes high and turns on T207, which again pulls down the reference voltage for IC204a. This means that the output voltage to the transmitter will be reduced.

The current limiter is made slow with C226 to pre vent the variation from the modulation of the transmitter. lt cannot protect the output transistors or rectifier against short circuit of the output.

IC206 delivers a regulated 8V supply for the voltage regulator and current limiter circuit.

The 28V output is regulated by IC205 and is adjusted with R271. Transistor T208 turns on and the current flows through L504 to C231 and the load.

The voltage increases at pin 4 and IC205 turns off T208. The current flows continuously in L504 and back through D214. The voltage will decrease at pin 4 and IC205 turns on T208 again. The regulator is then self oscillating. R268 forms a current limiter.

T301

+ BAT

}101/32 ~ }1DJ!33

-

0/ STAPT J PUI

HOI

3.4. 	BLOWER CONVERTER

When PU I is started up the oscillator IC301 is always running. The output stage and also the blowers are switched on/off by means of IC30l and IC303.

The supply for the blower converter control circuit is regulated to 12V by means of R322 and D301.

The frequency of the oscillator IC301 is 120 Hz and is determined by means of IC301, C303, R305, R306 and R307. The frequency is adjusted with R305. The output pulses from IC301 pin 3are led to IC303 pin 2 and to T303 which inverts the signal before it is led to IC302 pin 3, which produces a square wave signal of 60 Hz to the output stage.

In order to avoid the simultaneous conducting of all four output transistors T506 - T509, a dead time is performed with IC303, which pulls

down the base of T304 and T305 via D302 and D303. This will block the output stage.

If IC301 is not activated, IC303 pin 4 is grounded via R313 and the dead time is 100 % and then the output stage is off. If IC303 pin 4 is pulled high via IC301, the output pulse from IC303 pin 3 is 7.5 ms, which is adjusted with R311 and the blowers are running.

The output stage is a bridge coupled push-pull stage and consists of four Darlington power transistors T506 - T509 driven by the transistors T304 and T305.

By means of IC301 the blowers can be switched on in three ways. IC301 is always biased with 22V from PUI.

If temperature in the whole power supply exceeds 550C, the thermal breaker TB501 is switched on and starts the blowers until the temperature

is approx. 40ºC.

When transmitter is keyed, diode D306 is grounded via handset key and the blowers are running

The temperature protection unit in the transmitter unit breaks the connection between +22V and supply block terminal at Jl02. This turns off T301 which again turns on IC301 and thus also the blowers.

CONTENTS:

- COMPONENT LOCATIONS:

4.1 	CAPACITOR UNIT (MODULE 100)

4.2 	PU I AND PU II CONTROL UNIT (MODULE 200)

4.3 	BLOWER CONVERTER (MODULE 300)

4.4 	INPUT FILTER AND FUSES (MODULE 400)

4.5 	CHASSIS MOUNTING (MODULE 500)

4.9 	DISASSEMBLING FROM Tl130

4.10 	MAIN DIAGRAM

Component location

WIRECOLOURS

0 = BLACK

1 = BROWN

2 = RED

3 = ORANGE

4 = YELLOW

5 = GREEN

6 = BLUE

7 = VILOLET

8 = GREY

9 = WHITE

Ex. BROWN / WHITE = 19

4.1. CAPACITOR UNIT (MODULE 100)

  - CHASSIS MOUNTING (MODULE 500)

  - PU I AND PU II CONTROL UNIT (MODULE 200)

WIRECOLOURS

0 = BLACK

1 = BROWN

2 = RED

3 = ORANGE

4 = YELLOW

5 = GREEN

6 = BLUE

7 = VILOLET

8 = GREY

9 = WHITE

Ex. BROWN / WHITE = 19

WIRECOLOURS

0 = BLACK

1 = BROWN

2 = RED

3 = ORANGE

4 = YELLOW

5 = GREEN

6 = BLUE

7 = VILOLET

8 = GREY

9 = WHITE

Ex. BROWN / WHITE = 19

  - INPUT FILTER AND FUSES (MODULE 400)

  - BLOWER CONVERTER (MODULE 300)

WIRECOLOURS

0	=	BLACK

1	=	BROWN

2	=	RED

3	=	ORANGE

4	=	YELLOW

5	=	GREEN

6	=	BLUE

7	=	VILOLET

8	=	GREY

9	=	WHITE

Ex. BROWN / WHITE = 19

  - DISASSEMBLING FROM T1130

.;'

CONTENTS:

5 	PART LISTS

Capacitor unit N1407	 	(100)	1/1

Symbol Discripion 			Manufact.

C101	Capacitor,	electrolytic	2200 uF -10/+50%	 40V	ROE 		EG 03 MG 422G

C102 	Capacitor,	electrolytic	2200 uF -10/+50%	 40V	ROE 		EG 03 MG 422G

C103 	Capacitor,	electrolytic	2200 uF -10/+50%	 40V	ROE 		EG 03 MG 422G

C104 	Capacitor,	electrolytic	2200 uF -10/+50%	 40V	ROE 		EG 03 MG 422G

C105 	Capacitor,	electrolytic	2200 uF -10/+50%	 40V	ROE 		EG 03 MG 422G

C106 	Capacitor,	electrolytic	2200 uF -10/+50%	 40V	ROE 		EG 03 MG 422G

Rl01 	Resistor 	S.P. 	TL379

PU I and PU II unit N1407				(200)	1/5

Symbol	Description					Manufact.

C201 	capacitor, polyester 	 100 nF	+/-10% 	100V	Siemens 	B32510-D1104-K

C202 	capacitor, polyester 	 100 nF	+/-10% 	100V	Siemens 	B3251O-D1104-K

C203 	capacitor, polyester 	 100 nF	+/-10% 	100V	Siemens 	B32510-D1104-K

C204 	capacitor, electrolytic 	  22 uF	+/-20% 	 25V	ROE 	EKI 00 AA 222E

C205 	capacitor, polyester 	 100 nF	+/-20% 	400V	ERO 	MKT 1822-410-406

C206 	capacitor, polyester 	  22 nF	+/-5% 	400V	ERO 	MKT 1822-322-404

C207 	capacitor, polyester 	 330 nF	+/-20% 	 63V	ERO 	MKT 1818-433-066

C208 	capacitor, polyester 	  22 nF	+/-5% 	400V	ERO 	MKT 1822-322-404

C209 	capacitor, electrolytic 	  22 uF	+/-20% 	 25V	ROE 	EKI 00 AA 222E

C210 	capacitor, polyester 	 100 nF	+/-10% 	100V	Siemens 	B32510-Dl104-K

C211 	capacitor, polyester 	 220 nF	+/-20% 	100V	ERO 	MKT 1822-422-016

C212 	capacitor, polyester 	 220 nF	+/-20% 	100V	ERO 	MKT 1822-422-016

C213 	capacitor, polyester 	 220 nF	+/-20% 	100V	ERO 	MKT 1822-422-016

C214 	capacitor, electrolytic 	 470 uF	-10/+50% 	 50V	ROE 	EKM 00 JG 347H

C215 	capacitor, electrolytic 	2200 uF	-10/+50% 	 16V	ROE 	EKM 00 JG 422D

C216 	capacitor, electrolytic 	 100 uF	-10/+50% 	 63V	ROE 	EKM 00 DE 310J

C217 	capacitor, electrolytic 	  10 uF	+/-20% 	 35V	ROE 	EKI 00 AA 210F

C218 	capacitor, polyester 	  47 nF	+/-10% 	250V	Siemens 	B32510-D3473-K

C219 	capacitor, polyester 	 100 nF	+/-10% 	100V	Siemens 	B32510-D1104-K

C220 	capacitor, electrolytic 	 220 uf	-10/+50% 	 25V	ROE 	EKR 00 FE J22E

C221 	capacitor, electrolytic 	 100 uF	-10/+50% 	 16V	ROE 	EKM 00 CC 3100

C222 	capacitor, electrolytic 	 220 uF	–10/+50% 	 25V	ROE 	EKR 00 FE 322E

C223 	capacitor, electrolytic 	1000 uF	-10/+50% 	 25V	ROE 	EKM 00 JG 410E

C224 	capacitor, ceramic 	  10 nF	-20/+00% 	 50V	KCK 	HE70 SJYF 103Z

C225 	capacitor, ceramic 	  10 nF	-20/+80% 	 50V	KCK 	HE70 SJYF 103Z

C226 	capacitor, polyester 	   1 uF	+/-10% 	100V	Siemens 	B32511-01105-K

C227 	capacitor, electrolytic 	  10 uF	+/-20% 	 35V	ROE 	EKI 00 AA 210F

C228 	capacitor, polyester 	 100 nF	+/- 10% 	100V	Siemens 	832510-01104-K

C229 	capacitor, polyester 	 220 nF	+/-10% 	100V	Siemens 	B32511-01224-K

C230 	capacitor, polyester 	 100 nF	+/-10% 	100V	Siemens 	B32S10-Dl104_K

C231 	capacitor, electrolytic 	 470 uF	-10/+50% 	 50V	ROE 	EKM 00 JG 347H

C232 	capacitor, polyester	 220 nF	+/-10% 	100V	Siemens 	B32511-D1224-K

PU I and PU II unit N1407	(200)	2/5

Symbol 	Description	 		Manufact.

D201 	Diode, zener 	12V +/-5% 	2.5W	Motorola 	MZP4742A

D202 	Diode 			Philips 	BAV21

D203 	Diode 			Motorola 	MR750

D204 	Diode 			Motorola	MR750

D205 	Diode bridge 			G. I. 	2 KBP 04

D206 	Diode, zener 	75V +/-5% 	2.5W	Motorola 	MZP4761A

D207 	Diode, zener 	75V +/-5% 	2.5W	Motorola 	MZP4761A

D208 	Diode 			Motorola 	1N4002

D209 	Diode 			Motorola 	1N4002

D210 	Diode 			Motorola 	MR850

D211 	Diode, zener 	2.7V +/-5%	0.5W	Motorola 	BZX83 C2V7

D212 	Diode			Philips 	BAV21

D213 	Diode, zener 	5.1V +/-5%	0.5W	Philips 	BZX79 C5V1

D214 	Diode 			Motorola 	MR850

IC201 	Integrated circuit 			Siemens 	TDA471SA

IC202 	Integrated circuit 			Motorola 	MC 17 23CP

IC203 	Integrated circuit 			Motorola 	MC 17 23 CP

IC204a	Integrated circuit 			National 	1/2 LM358N

IC204b	Integrated circuit 			National 	1/2 LM358N

IC205 	Integrated circuit 			Motorola 	MC1723CP

IC206 	Voltage regulator 	8V		Motorola 	MC78LOSACP

OC201 	Opto coupler 			Siemens 	CNY 17-2

PU I and PU II unit N1407 	(200) 	3/5

Symbol	Description 		Manufact.

R201 	Resistor 	470  ohm	+/-5% 	2.5W	Philips 	2322 192 34701

R202 	Resistor 	2.2 Kohm	+/-5% 	0.33W	Philips 	2322 211 13222

R203 	Resistor 	2.2 Kohm	+/-5% 	0.33W	Philips 	2322 211 13222

R204 	Resistor 	 18 Kohm	+/-5% 	0.33W	Philips 	2322 211 13183

R205 	Resistor 	1.8 Kohm	+/-5% 	0.33W	Philips 	2322 211 13182

R206 	Resistor 	1.5 Kohm	+/-5% 	O.33W	Philips 	2322 211 13152

R207 	Resistor 	1.5 Mohm	+/-5% 	0.33W	Philips 	2322 181 53155

R208 	Resistor 	6.8 Kohm	+/-5% 	0.33W 	Philips 	2322 211 13682

R209 	Resistor 	1.5 Kohm	+/-5% 	1.6W	Philips 	2322 191 31502

R210 	Resistor 	0.22 ohm	+/-10% 	4W	Philips 	2322 329 34227

R211 	Resistor 	0.22 ohm	+/-10% 	4W	Philips 	2322 329 34227

R212 	Resistor 	1.5 Kohm	+/-5% 	1.6W	Philips 	2322 191 31502

R213 	Resistor 	100  ohm	+/-5% 	0.33W	Philips 	2322 211 13101

R214 	Resistor 	100  ohm	+/-5% 	0.33W	Philips 	2322 211 13101

R215 	Resistor 	8.2 Kohm	+/-5% 	0.33W	Philips 	2322 211 13822

R216 	Resistor 	  l Kohm	+/-5% 	0.33W	Philips 	2322 211 13102

R217 	Resistor 	1.5 Kohm	+/-5% 	0.5W	Philips 	2322 212 13152

R218 	Resistor 	1.5 Kohm	+/-5% 	0.5W	Philips 	2322 212 13152

R219 	Resistor 	  1 Kohm	+/-5% 	0.33W	Philips 	2322 211 13102

R220 	Resistor 	  l Kohm	+/-5% 	0.33W	Philips 	2322 211 13102

R221 	Resistor 	150  ohm	+/-5% 	0.33W	Philips 	2322 211 13151

R222 	Resistor 	1.5 Kohm	+/-5% 	0.33W	Philips 	2322 211 13152

R223 	Resistor 	2.7 Kohm	+/-5% 	0.33W	Philips 	2322 211 13272

R224 	Resistor 	 27  ohm	+/-5% 	2.5W	Philips 	2322 192 32709

R225 	Resistor 	100  ohm	+/-5% 	2.5W	Philips 	2322 192 31001

R226 	Resistor 	560  ohm	+/-5% 	0.33W	Philips 	2322 211 13561

R227 	Resistor	  1 Mohm	+/-5% 	0.33W	Philips 	2322 211 13105

R228 	Resistor 	 56  ohm	+/-5% 	0.33W	Philips 	2322 211 13569

R229 	Resistor 	4.7 Kohm	+/-5% 	0.33W	Philips 	2322 211 13472

R230 	Resistor 	2.7 Kohm	+/-5% 	0.33W	Philips 	2322 211 13272

R231 	Resistor 	0.11 ohm	+/-10% 	5W	Philips 	2322 329 35117

R232 	Resistor 	0.11 ohm	+/-10% 	5W	Philips 	2322 329 35117

R233 	Resistor	0.11 ohm	+/-10% 	5W	Philips 	2322 329 35117

R234 	Resistor 	  1 Kohm	+/-5% 	0.33W	Philips 	2322 211 13102

R235 	Resistor 	680  ohm	+/-5% 	0.33W	Philips 	2322 211 13681

R236 	Resistor 	8.2 Kohm	+/-5% 	0.33W	Philips 	2322 211 13822

R237 	Resistor 	2.7 Kohm	+/-5% 	0.33W	Philips 	2322 211 13272

R238 	Pre.Set.Potentiometer 	  1 Kohm	+/-10% 		Noble 	TM8 KV2-1S-1K-l0%

R239 	Resistor 	560 Kohm	+/-5% 	0.33W	Philips 	2322 211 13561

R240 	Resistor 	3.3 Kohm	+/-5% 	0.33W	Philips 	2322 211 13332

R241 	Pre.Set.Potentiometer 	 10 Kohm	+/-20% 		Noble 	TM8 KV2-1S-10K

R242 	Resistor 	2.7 Kohm	+/-5% 	0.33W	Philips 	2322 211 13272

R243	Pre.set.Potentiometer 	 l0 Kohm	+/-20%		Noble 	TM8 KV2-13-10K

R244 	Resistor 	4.7 Kohm	+/-5% 	0.33W	Philips 	2322 211 13472

R245 	Resistor 	220  ohm	+/-5% 	0.33W	Philips 	2322 211 13221

R246 	Resistor 	 56 Kohm	+/-5% 	0.33W	Philips 	2322 211 13563

R247 	Resistor 	 56 Kohm	+/-5% 	0.33W	Philips 	2322 211 13563

R248 	Resistor 	2.2 Kohm	+/-5% 	0.33W	Philips 	2322 211 13222

R249 	Resistor 	 18 Kohm	+/-5%	0.33W	Philips 	2322 211 13183

R250 	Resistor 	560 Kohm	+/-5% 	0.33W	Philips 	2322 211 13564

R251 	Resistor 	 15 Kohm	+/-5% 	0.33W	Philips 	2322 211 13153

R252 	Resistor 	 18 Kohm	+/-5% 	0.33W	Philips 	2322 211 13183

R253 	Resistor 	2.2 Kohm	+/-5% 	0.33W	Philips 	2322 211 13222

R254 	Resistor 	390  ohm	+/-5% 	0.33W	Philips 	2322 211 13391

R255 	Resistor 	 22  ohm	+/-5% 	0.33W	Philips 	2322 211 13229

R256 	Resistor 	  1 Kohm	+/-5%	0.33W	Philips 	2322 211 13102

R257 	Resistor 	1.5 Kohm	+/-5% 	0.33W	Philips 	2322 211 13152

R258 	Resistor 	2.7 Kohm	+/-5% 	0.33W	Philips 	2322 211 13272

R259 	Pre.Set.Potentiometer 	 22 Kohm	+/-20%		Noble 	TM8 KV2-13-22K

R260 	Resistor 	 56 ohm	+/-5% 	0.5W	Philips 	2322 212 13569

PU I and PU II unit N1407 			(200) 	4/5

Symbol 	Description			 	Manufact.

R261 	Resistor 	680 ohm	+/-5% 	0.33W	Philips 	2322 211 13681

R262 	Resistor 	  1Mohm	+/-5% 	0.33W	Philips 	2322 211 13105

R263 	Resistor 	8.2Kohm	+/-5% 	0.33W	Philips 	2322 211 13822

R264 	Resistor 	100kohm	+/-5% 	0.33W	Philips 	2322 211 13104

R265 	Resistor 	 15Kohm	+/-5% 	0.33W	Philips 	2322 211 13153

R266 	Resistor 	 51Kohm	+/-5% 	0.33W	Philips 	2322 211 13513

R267 	Resistor 	  1Kohm	+/-5% 	0.33W	Philips 	2322 211 13102

R268 	Resistor 	0.11ohm	+/-10% 	5W	Philips 	2322 329 35117

R269 	Resistor 	470 ohm	+/-5% 	0.33W	Philips 	2322 211 13471

R270 	Resistor 	3.9Kohm	+/-5% 	0.33W	Philips 	2322 211 13392

R271 	Pre.set.Potentiometer 	  1Kohm	+/-10% 		Noble	TM8 KV2-1S-1K-10%

R272 	Resistor 	620 ohm	+/-5% 	0.33W	Philips 	2322 211 13621

R273 	Resistor 	 68Kohm	+/-5% 	0.33W	Philips 	2322 211 13683

RE201 	Relay 				PASI 	KS/U-BV998

RE202 	Relay 				PASI 	KS/U-BV998

RE203 	Relay 				PASI 	KS/U-BV998

RE204 	Relay 				PASI 	KS/U-BV998

RE205 	Relay 				PASI 	KS/U-BV998

T201 	Transistor 			Philips	BD650

T202 	Transistor 			Philips	BD650

T203 	Transistor 			Motorola	BDX34B

T204 	Transistor 			Philips	BD138

T205 	Transistor 			Philips	BC640

T206 	Transistor 			Philips	BC639

T207 	Transistor 			Philips	BC338

T208 	Transistor 			Motorola	BDX34B

TR201	Transformer 			Tradania	TD3435

TR202	Transformer 			Tradania	TD4265

TR203	Transformer 			Tradania	TD4265

Blower converter N 1407	(300)		1/2

Symbol Description				Manufact.

C301 	Capacitor, polyester 	100 nF	+/-10% 	100V	Siemens 	B32510-Dl104-K

C302 	Capacitor, polyester 	 47 uF	+/-20% 	 25V	ROE 	EK1 00 BB 247E

C303 	Capacitor, polyester 	220 nF	+/- 5% 	 63V	ERO 	MKT 1818 422 064

C304 	Capacitor, polyester 	 10 nF	+/-1O% 	400V	Siemens 	B32510-D6103-K

C305 	Capacitor, polyester 	 10 nF	+/-1O% 	400V	Siemens 	B32510-D6103-K

C306 	Capacitor, polyester 	220 nF	+/- 5% 	 63V	ERO 	MKT 1818 422 064

C307 	Capacitor, polyester 	 10 nF	+/-10% 	400V	Siemens 	B32510-D6103-K

1C301 Integrated circuit 				Motorola 	MC1455 P1

1C302 Integrated circuit 				Motorola 	MC14013 BCP

1C303 Integrated circuit 				Motorola 	MC1455 P1

D301 	Diode, zener- 	12V	+/- 5% 	2.5W Motorola 	MZP4742A

D302 	Diode 				Motorola 	lN4148

D303 	Diode 				Motorola 	lN4148

D304 	Diode 				Motorola 	lN4148

D305 	Diode 				Motorola 	lN4148

D306 	Diode 				Philips 	BAV21

OC301 Opto coupler 				Siemens 	CNY 17-2

R301 	Resistor 	 33 Kohm	+/-5% 	0.33W	Philips 	2322 211 13333

R302 	Resistor	 10 Kohm	+/-5% 	0.33W	Philips 	2322 211 13103

R303 	Resistor	 33 Kohm	+/-5% 	0.33W	Philips 	2322 211 13333

R304 	Resistor	1.5 Kohm	+/-5% 	0.5W	Philips 	2322 212 13152

R305 	Pre.Set. Potentiometer	 22 Kohm	+/-10% 		Noble 	TM8 KV2-1S-22K-l0%

R306 	Resistor	 33 Kohm	+/-5% 	0.33W	Philips 	2322 211 13333

R307 	Resistor	5.6 Kohm	+/-5% 	0.33W	Philips 	2322 211 13562

R308 	Resistor	100 Kohm	+/-5% 	0.33W	Philips 	2322 211 13104

R309 	Resistor 	1.5 Kohm	+/-5% 	0.33W	Philips 	2322 211 13152

R310 	Resistor	 10 Kohm	+/-5% 	0.33W	Philips 	2322 211 13103

H311 	Pre.Set.Potentiometer 	 22 Kohm	+/-10% 		Noble 	TM8 KV2-1S-22K-l0%

R312 	Resistor 	 18 Kohm	+/-5% 	0.33W	Philips 	2322 211 13183

R313 	Resistor	  1 Kohm	+/-5% 	0.33W	Philips 	2322 211 13102

R314 	Resistor 	3.3 Kohm	+/-5% 	0.33W	Philips 	2322 211 13332

R315 	Resistor 	8.2 Kohm	+/-5% 	0.33W	Philips 	2322 211 13822

R316 	Resistor 	470 Kohm	+/-5% 	0.33W	Philips 	2322 211 13474

R317 	Resistor 	8.2 Kohm	+/-5% 	0.33W	Philips 	2322 211 13822

R318 	Resistor 	470 Kohm	+/-5% 	0.33W	Philips 	2322 211 13474

R319 	Resistor 	  1 Kohm	+/-5% 	0.33W	Philips 	2322 211 13102

R320 	Resistor 	470  ohm	+/-5% 	1.6W	Philips 	2322 191 34701

R321 	Resistor 	  1 Kohm	+/-5% 	0.33W	Philips 	2322 211 13102

R322 	Resistor 	330  ohm	+/-5% 	2.5W	Philips 	2322 192 33301

R323 	Resistor 	  1 Kohm	+/-5% 	0.33W	Philips 	2322 211 13102

R324 	Resistor 	470  ohm	+/-5% 	1.6W	Philips 	2322 191 34701

R325 	Resistor 	  1 Kohm	+/-5% 	0.33W	Philips 	2322 211 13102

T301	Transistor				Philips 	BC338

T302 	Transistor 				Philips 	BC3J8

T303 	Transistor 				Philips 	BC548

T304 	Transistor 				Motorola 	MPS-AO6

T305 	Transistor 				Motorola 	MPS-AO6

Input filter and fuses N1407 			(400) 	1/1

Symbol	Description				Manufact.

C401 	Capacitor, polyester	2.2uF	 +/-10% 	100V	Siemens 	B32512-E1225-K

C402 	Capacitor, polyester	2.2uF	 +/-10% 	100V	Siemens 	B32512-E1225-K

C403 	Capacitor, polyester	2.2uF	 +/-10% 	100V	Siemens 	B32512-E 1225-K

C404 	Capacitor, polyester	2.2uF	 +/-10% 	100V	Siemens 	B32512-E1225-K

C405 	Capacitor, polyester	2.2uF	 +/-10% 	100V	Siemens 	B32512-E1225-K

C406 	Capacitor, polyester	2.2uF	 +/-10% 	100V	Siemens 	B32512-E1225-K

C407 	Capacitor, polyester	1uF	 +/-20% 	100V	ERO 	MKT 1822-510-016

C408 	Capacitor, polyester	1uF	 +/-20% 	100V	ERO 	MKT 1822-510-016

C409 	Capacitor, electrolytic	2200uF	 -10/+50%	 40V	ROE 	EG 00 MG 422G

C410 	Capacitor, electrolytic 	2200uF	 -10/+50%	 40V	ROE 	EG 00 MG 422G

D40l 	Diode 				Motorola 	MR750

D402 	Diode 				Motorola 	MR750

F401 	Fuse 	10A 3.6 x 32mm		Wickmann	311010

F402 	Fuse 	35A 3.6 x 32mm		Wickmann	311035

L401 	Coil 				S. P. 	TL354

24V POWER SUPPLY + T1130 N1407 			(500) 	1/1

Symbol	Description 				Manufact.

C501 	Capacitor, polycarbonate	 470 nF	+/-10%	250V	ERO	MKC 1860-447-255

C502 	Capacitor, polycarbonate	 470 nF	+/-10% 	250V	ERO	MKC 1860-447-255

C502 	Capacitor, electrolytic	4700 uF	-10/+30%	 40V	FRAKO	EBA 4700-40 isol

C504 	Capacitor, polycarbonate	 470 nF	+/-10% 	250V	ERO	MKC 1860-447-255

D501 	Diode bridge				G.I. 	KBPC6T 005

D502 	Diode bridge				G.I. 	KBPC6T 005

D503 	Diode, zener	  75V :5% 	  5W	Motorola	lN5374B

D504 	Diode, zener	  75V :5% 	  5W	Motorola	lN5347B

D505 	Diode, bridge				Matorola	BYW61

L501 	Choke		 		Tradania	TD4573

L502 	Choke		 		Tradania	TD4573

L503 	Choke		 		Tradania	TD4240

L504 	Choke		 		Tradania	TD4573

LSOS 	Choke		 		S. P . 	TL383

R501 	Resistor 	220 ohm : 10% 	10W	ARCOL	HS10, 220, 10%

R502 	Resistor 	2.7 ohm :  5% 	2.5W	Philips	2322 192 32708

R503 	Resistor 	2.7 ohm :  5% 	2.5W	Philips	2322 192 32708

R504 	Resistor 	2.7 ohm :  5% 	2.5W	Philips	2322 192 32708

R505 	Resistor, varistor 	S14K50		 	Siemens	Q69-X3135

T501 	Transistor (Matched pair)				Motorola	MJ802

T502 	Transistor (Matched pair)				Motorola	MJ802

T503 	Transistor				Motorola	BD808

T504 	Transistor (Matched pair)				Sescossem	BUV19

T505 	Transistor (Matched pair)				Sescossem	BUV19

T506 	Transistor		 		Philips	BD650

T507 	Transistor		 		Philips	BD649

T508 	Transistor		 		Philips	BD650

T509 	Transistor		 		Philips	BD649

TB501 	Thermostat	ON  55 +/-4ºC		Elmwood 	22455R-21-923

OFF 40 +/-4ºC

TR501 	Transformer				Tradania 	TD335

TR502 	Transformer				Tradania 	TD324