NOSaprs 1.12

APRS Server & Internet Gateway for TNOS and JNOS

Designed & Coded by Maiko Langelaar, VE4KLM

during the period of April 2001 to March 2002

Thursday, April 4, 2002

Table of Contents

Title page								1

Table of contents (this page)						2

What is NOSaprs							3

Some of the Features							3

How does it work							4

What does NOSaprs hear						4

Routing - By Message Recipient						5

Routing - By which interface station was last heard on			5

Opting out of IGATE							5

Flexible Email Server							5

Simple Message Center							6

NOSaprs Log File							6

Status and Position Broadcasts						6

Tracking APRS Users - the APRS Heard Table				6

Status and other information – using the tcp/ip finger utility		7

Status and other Information – via port 14501 and a web browser		7

Typical entries for the ‘autoexec.nos’ file					9

Most of the NOSaprs commands					9

The rest of the NOSaprs commands					13

The APRS Client Connection (44825) Server				13

Miscellaneous stuff and other notes					14

What is NOSaprs

NOSaprs is an APRS Server and Internet Gateway (IGATE) that is integrated into the NOS itself. Those that are already running JNOS or TNOS internet/packet gateways can easily start to support their local APRS community by simply getting a NOS binary that contains the NOSaprs services, and then adding a few extra lines to the autoexec.nos file. Use an existing RF port (zoo channel), add a new one for 144.390, add multiple RF ports on different frequencies, use a 9600 baud port, use AXIP or AXUDP to get APRS data from other internet gateways around the world, do whatever is possible with your NOS environment. I believe that integrating APRS services into NOS opens up a wide variety of options to the APRS world.

Some of the Features

- Intercepts APRS data heard on all NOS ax25 interfaces, and forwards the data to the APRS internet system, which in turn allows the data to appear on the maps of APRS client software throughout the world, as well on internet APRS databases like www.findu.com, www.canaprs.net, and others.

- Allows APRS users on any NOS ax25 interface, to exchange messages with other APRS users on remote APRS networks only reachable through the APRS internet system, using the 3rd party messaging system.

- Tracks APRS users heard on all NOS ax25 interfaces. The system can track callsign, DTI, which ax25 interface the frame was heard on, and the unproto (digi) path of the frame if any digis were used. This information is available by finger aprsstat@machine or by web browser at http://machine:14501.

- When forwarding APRS data to the APRS internet system, a Point of Entry (POE) callsign is inserted into the source path header, allowing other systems to identify your machine as the one which injected the APRS packets into the APRS internet system. This can be disabled by the sysop if desired.

- Intercepts MIC-E packets, decodes them, and forwards the information to the APRS internet system.

- Supports routing by message recipient, allowing APRS users on conventional packet networks to participate in the APRS scheme of things. Users several nodes away can now join in on the APRS fun without the need for WIDE, or RELAY, or APRS digipeaters.

- APRS Email Server. Email can be handled locally by NOS itself, instead of being sent out to the APRS internet system for processing. Any APRS user can send internet email, simply by sending an APRS message to the recipient callsign, ‘EMAIL’.

- APRS users can opt out of the IGATE by specifying RFONLY or NOGATE in the unproto path, that way their packets are not forwarded to the APRS internet system.

- Message Center, allowing the NOS sysop, and the general PBBS user, to do message passing with any other APRS user, either on a remote APRS network only reachable through the APRS internet system, or through one of the NOS ax25 interfaces for local message passing. Some restrictions apply.

- An HTML based status page, similar in concept to what APRSD provides, using TCP port 14501. A sample screen shot of this HTML based page, is shown on the opening page of this documentation. A text based version is also available using the tcp/ip ‘finger’ utility. See the ‘Tracks APRS users’ feature mentioned earlier on this page.

- Flexible broadcast options. NOSaprs has separate broadcast parameters for the internet side and the RF side. Each side has it’s own set of parameters, in which you can configure the broadcast interval, status text, and position information. If you don’t want broadcasts at all, they can be shutoff if desired.

- Responds to ?PING? or ?APRST requests from APRS internet system and from the RF side. You can also send a message to NOSaprs, which is logged to a file (ie, /nos/spool/aprs/aprs.txt). All messages to and from NOSaprs logon callsign are logged (including sysop and PBBS message center traffic).

- Accept's connections from other programs like UI-View, WINaprs, and other NOSaprs stations so that they can forward APRS data to us and not get the firehose of data sent back to them.This is also ideal for those systems that have very low speed (or low bandwidth) connections, and simply want to advertise their position and other information to the world and not hear back from the world.

This last feature is experimental, and is actually disabled (by default) in this release, due in part to some inconsistent problems which I have experienced while running it. Also, I am redesigning it. That does not mean you can’t use it.  Just edit the aprs.h header file and define the 44825 server, then recompile.

- Other features are added as time permits.

How does it work

NOSaprs has the minimum functionality required to be an APRS Internet Gateway (IGATE), so it should be configured to connect to TCP port 1314 (the message port) of any APRSD type server that is part of the APRS internet system. At startup time, NOSaprs will go through a list of servers, connecting to the first one it can. Once connected, it will continuously receive messages from the server, and route those messages out a default NOS ax25 interface (configured by the ‘aprs port xxx’). There is a filter feature (configured using the ‘aprs fwd2rf xxx’ command) so that the sysop can control which messages are allowed to go out, and which are not (based on the callsign pattern of the message recipient). APRS frames received on all NOS ax25 interfaces are sent to the APRS internet system, using the same connection we receive messages on.

A Very Important Note : When connecting to a remote server, there is no point connecting to any other port than port 1314. The reason is simply that NOSaprs only processes direct user messages from the APRS internet system. Everything else is presently ignored. Connecting to other (and much busier) ports will only load down NOS for nothing, and add to the cpu utilization.

Another important note : Keep in mind that NOSaprs only connects to 1 host server at any time, unlike other APRS servers that can be configured for multiple connections to different host servers, for the sake of redundancy, etc.

I am working on supporting the broadcast of status and position information received from the APRS internet system, meaning a different port will have to be used. Port 1314 only provides messages, and not status and position information. I have not implemented this yet, but it will appear in a near future version.

What does NOSaprs hear

NOSaprs will hear any APRS packets that have a destination call of ‘BEACON’, ‘ID’, ‘EMAIL’, or any callsign starting with the letters ‘AP’, or any callsign that matches the APRS MIC-E callsign requirement. The number of digipeaters in the packet header, or whether any of the digipeaters were used to repeat the packets, is of no importance. NOSaprs will hear it all, and process it accordingly. I have fairly strong validation rules in place, so not everything heard is sent to the APRS internet system. For example, 3rd party packets heard on any of the NOS ax25 ports, are NOT forwarded to the internet system, since they’re not supposed to be.

Routing - by Message Recipient

By default, all 3rd party messages are sent out the local RF port, using the unproto path ‘APZ112’, which I use instead of the usual ‘APRS’ because of the fact that this is experimental software. If you want to change the unproto path, simply use the NOS ax25 route add command to do so. For example, if you would rather have NOSaprs send it’s APRS frames as APZ112 via WIDE, then you might have a command similar to the following :

ax25  route  add  APZ112  ax0  WIDE

That is one of the nice things about writing software for NOS. I did not have to write special route commands specific to the APRS application. The standard NOS functions and commands fit in perfectly.

What is particularily interesting about NOSaprs is it’s ability to route a 3rd party message by recipient !!!

If you program a route to a specific callsign, using the NOS ax25 route add command, then NOSaprs will actually route any 3rd party messages for that callsign (recipient) using the the pre-programmed route for that callsign, effectively overriding the default unproto path. If a message comes in, NOSaprs will know exactly where to digipeat the frame based on the recipient of the message. That’s a powerful option in my opinion and provides some degree of flexibility for users that want to play with APRS on conventional packet networks.

Notes : routing by recipient is purely an option, and is not forced on the sysop.

Routing – By which Interface station was last heard on

As of release 1.09, NOSaprs can route messages (coming from the APRS internet system) out the interface that the recipient station was last heard on, instead of through the default APRS interface. Of course, if the station was not heard, then the message is gated out the default interface. Because of this feature, users on any NOS ax25 interface should now be able to messaging with other APRS stations via the APRS internet system. Routing between NOS ax25 interfaces on the same machine is not implemented (yet).

Opting out of the IGATE

Some users may not want any of their APRS frames to be sent to the APRS internet system. If those users wish to opt out of using the IGATE, they can insert the callsign NOGATE or RFONLY into their unproto path. They will still appear on the APRS heard table, but their traffic will not be gated to the internet.

Flexible Email Server

Any APRS user on the RF side or on any of the AXIP or AXUDP wormholes can send a simple one line message to any internet email account, or to any local BBS or mailbox account. All you have to do is send a one line message to the ‘EMAIL’ recipient callsign. The format of the 1 line message is best illustrated with some examples :

The following example will send a message to my internet email address, maiko@langelaar.net.

maiko@langelaar.net hi Maiko, this is a test

The  example below will send a message to my BBS account on our local TNOS box.

ve4klm hi Maiko, this is a test message

The first field (no spaces) is interpreted as the email address to which you want the message sent. The remainder of the information is the body of the message. How and where it gets sent is effected by how you have your NOS rewrite file configured. Keep that in mind when offering this service.

NOSaprs can be configured to handle email requests in 3 ways :

- pass all request to the APRS internet system for processing.

- ignore (block) all email requests.

- handle email requests locally (use the built in NOS smtp services).

Yes you heard that right. NOS has it’s own built in SMTP services. The default configuration is to have NOS handle the email request locally. This is a powerful feature, because the NOS rewrite rules are automatically implemented with any built in mail functions. You can technically have an email request forwarded to a local FBB bbs system, instead of to the internet.

Notes : if NOS itself is handling any email requests, they are never sent to the APRS internet system. Any requests from the APRS internet system are NOT processed for obvious reasons, although they easily could be, it’s just one line in the source code that prevents this .

Simple Message Center

NOSaprs features a simple, but possibly useful APRS Message Center of sorts, for both the PBBS user and the sysop (from the NOS console). I've tried to give it a *converse* type of look. This is a start, the commands are very limited, etc. The current release requires the PBBS user to have TELNET permission in order to use it. This is experimental, so don't expect too much. To activate the message center, just use the command 'aprsc' from either the PBBS prompt or the NOS console. Some restrictions apply - it can not be run from within SYSOP mode, and PBBS users that have the same callsign as the NOSaprs server itself are not allowed to use it from the PBBS, they must use it from the NOS console instead, due to technical reasons I won’t get into.

The message center only works with stations reachable through the APRS internet system at this time. You can't use it to talk with stations on the local RF ports. That’s on my todo list. Also, any incoming messages that have a message id are only acked once, unlike the various Windows clients out there that can send multiple acks. That to is on my list of things to complete.

NOSaprs Log File

The APRS Server has it's own log file. APRS related events and information are now kept out of the main TNOS log. No point filling up the main log with junk. Add the following command into your autoexec.nos :

aprs log /nos/spool/log/aprslog.txt

Make sure it comes before ANY other 'aprs' type command if you want to use it.

Status and Position Broadcasts

At regular intervals, NOSaprs will broadcast Status and Position text to both the APRS internet system and to the default ax25 interface. As of release 1.10, you can now configure separate and independent broadcasts for the APRS internet system side and the RF side. In previous releases, the same Status and Position info was sent everywhere and at a hardcoded interval of 30 minutes. Check the command summary later on in this document for full details.

Tracking APRS Users – the APRS Heard Table

NOSaprs lets you dynamically configure a heard table of any size. The heard table keeps track of APRS users heard on all ax25 interfaces, including types of DTI characters seen per callsign, which ax25 interface the station was heard on, and the unproto (digi) path if used. The DTI is short for Data Type Identifier, which defines the type of APRS information being passed. That way, the sysop or other users can see who is doing APRS on any of the ax25 interfaces, and what type of APRS traffic is involved. I’m thinking of adding an option in a future release, where tracking of the DTI can be disabled, since the heard table can get quite big even if there are only a small number of callsigns using several different APRS packet types.

Status and other information – using the tcp/ip finger utility

To display the APRS Heard Table as well as the status of the internet connect, one can use the finger command. For example, to show the APRS heard table and connect status for ve4umr.ampr.org, use :

finger aprsstat@ve4umr.ampr.org

and it will give you something like what is shown below :

[ve4umr.ampr.org]

APRS status at ve4umr.ampr.org

Ver: TNOSaprs 1.12, Uptime: 5:16:03:10, Call: VE4UMR-5, Interface: 430

Server connection:

Server            Port  Connected   Pkts In   Bytes     Pkts Out  Bytes

128.143.202.191      1314  0:01:50:00  3388      260032    12        662

* 209.114.215.25       1314  3:08:33:02  158451    12451159  661       38896

198.88.21.2          1314  0:03:34:43  10163     816118    33        1916

No APRS clients connected at this time

APRS traffic heard:

Callsign      DTI     Since     Pkts  Other Info (I=Interface, P=Path)

ve4bbs         !    0:00:10:24  331   -

ve4klm         !    0:00:05:28  127   P ve4pkt* ve4pkt-8*

ve4klm         >    0:00:05:27  120   P ve4pkt* ve4pkt-8*

ve4klm-4       :    5:06:09:27  10    P ve4pkt* ve4pkt-8*

ve7rxd         !    0:00:23:03  237   I tsi, P kaipgu*

Please refer to the next section, for more information about what the above data means.

Notes : the * character beside a particular server entry indicates the server to which NOSaprs is currently connected with. If you don't see the * character at all, it means that NOSaprs is not currently connected to any of the servers in the list. It will try every 30 seconds to make a connection. What is particularily interesting about the above, is the fact that the ve4umr TNOS box is receiving APRS frames over AXIP wormholes that it has with several other machines around the world .

Status and other Information – via port 14501 and a web browser

The information from the finger command can also be made available in the form of an HTML page. If you have NOSaprs configured to run the 14501 server, then you can use any web browser to get the tracked information discussed in the previous section. I added this feature because the APRSD status pages use this system, and there is a link to each connected user of the APRSD server. Now people can actually click on any of the NOSaprs users, and see a page of information. A sample shot of the NOSaprs 14501 screen is actually shown on the opening page of this documentation.

In order to provide this service, you need at minimum, to put ‘start 14501’ in your autoexec.nos file. In order for a user to access this page, they would use a link similar to the following :

http://ve4umr.ampr.org:14501

The 14501 page is split into 3 sections – General information, Host Connection, and APRS traffic heard.

General Information

| Machine | The internet Domain Name of the NOS machine running NOSaprs |
| --- | --- |
| Uptime | The length of time the entire NOS system has been up. |
| Callsign | The APRS callsign of NOSaprs - also used when logging into the APRS internet system server. |
| Interface | The default ax25 interface for gating of 3rd party messages. |
| Contact Info | The sysop’s email address. |

Host Connection

Remember, NOSaprs only connects to ONE host server, unlike other APRS servers that have can have multiple connections for the sake of redundancy, etc. With the 14501 page, the host that is currently connected to will appear highlighted in YELLOW. If there is no highlighted entry, it means no connects are currently active.

| Server | The host name or ip address of a remote APRSD type server. This is our connection into the APRS internet system. |
| --- | --- |
| Port | The TCP port to connect to on the remote server. Currently you must use 1314, which is the messaging port of the APRS type server. |
| Connected | The total time we have been connected to this server, or if we are no longer connected, this is the length of time we were last connected to the server for. The display format is DD:HH:MM:SS. |
| Pkts (Bytes) In | Total number of packets and bytes we have received from the APRS internet system. |
| Pkts (Bytes) Out | Total number of packets and bytes we have sent to the APRS internet system. |
| Since | The time that has elapsed since we last heard something from the remote server. The display format is DD:HH:MM:SS. This field only available on the 14501 page, not the finger info. |

APRS Traffic Heard

| Callsign | Callsign of the station originating the APRS packet. |
| --- | --- |
| DTI | The APRS Data Type Identifier of the packet. |
| Since | The time since the packet was heard. The display format is DD:HH:MM:SS. |
| Pkts | Number of packets heard for this station / DTI combination. |
| Other Info | If the packet was heard on an ax25 interface other than the designated (default) ax25 interface, then the name of the interface will appear here. If the packet header had any digipeaters, then they will be displayed here (digipeaters that repeated the packet will be marked with a ‘*’ character). I means interface, P means (digipeater or unproto) path. |

As of release 1.10, there is a browser link on all the servers listed in the Host Connection section. These links are by default configured to access port 14501 of that particular server. If the server does not have a information port, you can always just set the info port to 80 (standard website) instead of 14501. That way you could at least provide a club website or something to curious users. This feature is described in the NOSaprs command summary further on in this document, under the ‘aprs server add …’ subcommand.

Typical entries for the ‘autoexec.nos’ file

Below is an typical configuration for NOSaprs 1.12. It’s best to run all the aprs commands after everything else is configured and running. In other words, put all the aprs commands at the end of the file.

aprs log /nos/spool/log/aprss.log

aprs email "ve4klm@ve4umr.ampr.org"

aprs logon ve4umr-5

aprs server add 128.143.202.191 1314

aprs server add 209.114.215.25 1314

aprs hsize 15

aprs interface 430

aprs fwdtorf ve4 va4

aprs bc stat "Server - 441.05 Mhz – ve4klm@ve4umr.ampr.org"

aprs bc pos "4948.53NI09707.95W&"

aprs bc timer 60

aprs bc rftimer 30

aprs listen on

start 14501

Most of the NOSaprs commands

Most of  the NOS commands related to NOSaprs start with the word ‘aprs’ and are as follows.

1.0	aprs <subcommand>

The current subcommands for release 1.12 are - bc, dup, flags, fwdtorf, hsize, interface, internal, listen, lmsg, log, logon, imsg, server, email, and emailmode.

1.0.1	aprs bc <subcommand>

NOSaprs broadcasts status and position information at regular intervals. Use these commands to configure the content of the status and position broadcasts, as well as the interval (in minutes) of those broadcasts. It is important to note that you can configure separate and independent broadcasts for both the RF side and the APRS internet system side.

1.0.1.1	aprs bc [stat | pos | rfstat | rfpos] [status or position text]

This command lets you set the text of the status and position information that you want broadcast by the NOSaprs server. There are four possible options here :

- stat – status text to be broadcast to the APRS internet system side.

- pos – position text to be broadcast to the APRS internet system side.

- rfstat – status text to be sent out the default ax25 interface side.

- rfpos – position text to be sent out the default ax25 interface side.

If  you want the same text broadcast to both the APRS internet system side and RF, then you only need to define the stat and pos options. If the rfstat and rfpos options are not defined, then they will default to the values set for stat and pos options. No point having duplicate information in the autoexec.nos file.

Example :

aprs bc stat "IGATE - 441.050 Mhz - ve4klm@ve4umr.ampr.org"

aprs bc pos "4948.50N/09707.84W-"

Note : I use the ! Data Type Identifier for the position broadcasts, so MAKE SURE you do not change the format of the above information. Only change the numerical coordinates (degrees, not decimal) to suit your own position. Also, the status text above actually has the NOSaprs version prepended to it before it is sent out. So in actually fact what is broadcast to the internet and local RF port for this example, looks like this :

xNOSaprs 1.12 IGATE - 441.050 Mhz - ve4klm@ve4umr.ampr.org

You can switch off the version by using ‘aprs bc ver|rfver’ command, described a bit further on.

      - aprs bc [timer | rftimer] [interval in minutes]

This command lets you set the interval of the status and position broadcasts. You can configure a different interval for either the internet side or the RF side. If you don’t want a particular broadcast, then either set the interval to the value of zero, or don’t define the timer(s) in the autoexec.nos file. By default, the timers are off.

      - aprs bc [ver | rfver] [on | off]

This command simply flags whether you want the NOSaprs version information prepended to the status broadcast(s) or not. By default, the version is included.

1.0.2	aprs dup <subcommand>

NOSaprs contains code to check for duplicate packets from the RF side. Currently this is only used for email requests, to make sure duplicate emails are not sent out just because an email request or an ack to such a request gets lost along the path. The duplicate table uses message checksums and message age to keep track of duplicate APRS frames. At this time, there is only one subcommand, ‘show’.

1.0.2.1	aprs dup show

This simply displays the duplicate table.

1.0.3	aprs flags [+|-option1] [+|-option2] … [+|-optionN]

The flags subcommand is used to toggle all options that are either on or off. If you do not specify any options at all, then NOS will simply display the current state of all flags, for example :

/nos> aprs flags : -debug -badpackets -monitor +poe

The current options are summarized in subsections 1.0.3.1 to 1.0.3.4 just below.

      - debug

If you run into problems and are lucky enough to be able to reproduce the problems, then you can enable the debug option, which will write additional debug or trace information to the NOSaprs log file.

      - badpackets

If you want to track parsing errors, or want to know about APRS frames that had errors in them or did not meet the general requirements of the ‘latest’ APRS specification, then you can enable the badpacket option, which will write error messages to the NOSaprs log for any packets that had format ‘problems’. Depending on what kind of day the APRS internet system is having, this too has the potential to make the NOSaprs log grow in size quickly.

      - monitor

If you want to monitor the exact data you are receiving from the internet side, you can eable the monitor option. This is simply an extension to the debug facilities. Be careful with this option, as the information is currently dumped to the NOSaprs log file. If the internet traffic is high, the log file will fill up really fast.

      - poe

This option is used to enable or disable the injection of the Point of Entry (POE) callsign into the source path header when forwarding packets to the APRS internet system. By default, the POE is turned on. It should not cause any problems, since the method has been shown to be 100 percent backwards compatible with the existing APRS internet system. I only made this command available in the rare instance that the feature may need to be switched off, which I can’t see happening. The POE callsign allows one to trace which IGATE received an APRS frame, and forwarded it into the APRS internet system. If you want more information about the POE, then contact me, and we can talk about it some more. As of today, there is still no unanimous agreement on how it should be properly implemented. The fact that there is still no official APRS internet system (IS) specification has only added to the dilemna.

1.0.4	aprs fwdtorf [callsign and/or callsign pattern list]

This is a powerful command that HAD to be implemented for messaging. I sure as heck don't want every single message coming from the internet side going out our local RF port. I wanted the ability to choose which callsigns (or callsign patterns) would be allowed to do so. The best way to show this is by example :

aprs  fwdtorf  ve va k*6 k*4k

The above example will gate messages to the local RF port, for any messages where the message recipient has a callsign that begins with either ‘ve’, or ‘va’, or starts off with the pattern, ‘k*6’, where the ‘*’ character is a wildcard (meaning anything). Note the ‘*’ wildcard is for one character position. In other words, the pattern, ‘k*6’, will allow any callsigns beginning with ‘ka6’, or ‘kb6’, or ‘kp6’, etc. If you want to wildcard more than one position you would need multiple ‘*’ characters. For example, ‘**6’, would allow any call where the 3rd character from the start of the callsign had the digit 6 in that position. Putting a trailing ‘*’ is redundant and is not necessary (for example, ‘ve4*’ does no more than using just ‘ve4’ as the pattern).

Note: This command can be run on the fly, so subsequent instances of this command will simply delete the existing fwdtorf list, and create a new one to replace it.

1.0.5	aprs hsize <size>

This creates the APRS heard table. The size parameter simply configures the maximum number of callsign/DTI combinations that can be held in the table before the oldest entries start to get overwritten.

Note: This command can be run on the fly, so subsequent instances of this command will simply delete the old heard table, and create a new empty one (with a new size if desired) to replace it.

1.0.6	aprs interface <port>

NOSaprs needs to know which ax25 port to use when gating messages from the internet side to RF. If you want to use the interface labeled, ‘ax0’, then set the aprs port as follows :

aprs interface ax0

This is the so called default NOS ax25 interface that is referred to in other parts of this manual.

1.0.7	aprs internal <hostaddr>

This is an optional command that is only required if you have 2 public IP addresses in use and NOS is doing the encapsulation. Most systems now have 1 public IP address it seems, so Linux winds up doing the encapsulation instead of NOS. In this case, the ‘aprs internal’ entry is not required and should be left out.

1.0.8	aprs listen [on | off]

Start or stop the aprs server, or display whether it is on or off.

1.0.9	aprs logon <callsign>

Configure or display the callsign used by NOSaprs when logging into the remote APRS server. This is also the source callsign of the APRS unproto frames that go out the local NOS ax25 interfaces.

    - aprs log <pathname of file>

If you to turn on logging for the APRS Server, then use this command. For example :

aprs log /nos/spool/log/aprslog.txt

important : make sure the ‘aprs log’ command is the first aprs command that is run in the autoexec.nos file. Also, make sure the directory path specified in the ‘aprs log’ command exists. If the directory path does not exist, then it won’t be created for you either (sorry, an oversite on my part).

1.0.11  aprs [imsg | lmsg] <recipient callsign> <message (no spaces)>

These 2 commands are old and more or less have been replaced by the APRS message center feature. I originally developed these for debugging purposes and testing, but they could still be of use to people. If you want to send a message to another APRS station on a remote network via the APRS internet system then you could do something like the following :

aprs  imsg  n1uan “Hi Bob. Just Testing. 73 de Maiko”

If you wanted to do the same, but send it out the local ax25 interface, then use ‘lmsg’ instead of ‘imsg’.

1.0.12	aprs server <subcommand>

This aprs server can maintain a list of servers to connect to. If it is not able to connect to the first server on the list, it will try to connect to the second one in the list, and so on, and so on, if need be. You must have a minimum of one server configured or else the system stays off.

1.0.12.1  aprs server add <hostaddr> <tcp/ip port> <info port>

Add a server to the list of servers that Nos will try to connect with. Note that each server entry consists of a host and a corresponding tcp/ip port. The <info port> is optional and new to release 1.10. If it is not specified than the HTML link for the particular server assumes port 14501. If that particular server uses a different port then you can configure it using the optional <info port> parameter.  If the server has no 14501 type service, then you could always use the standard port 80 and link to the providers website for information.

1.0.12.2  aprs server delete <hostaddr>

Delete a server from the list.

1.0.12.3 aprs server kick

If for some reason you suspect a problem with the APRS internet system server to which NOSaprs is currently connected to (for example, it’s no longer sending you data or seems dead), then you can issue this command to deal with it, instead of waiting for NOSaprs to check for inactivity. NOSaprs will then terminate the current connection, and try the next server in the list. I wish I had this command months ago !!!

1.0.12.4  aprs server list

Display the full list of servers.

1.0.13	aprs email [email address]

Set the email address that will appear in the Contact Information section of the 14501 status page. If this is not set, then the 14501 page will simply show “None Provided”.

    - aprs emailmode [passthru | local | block]

This command configures how you want NOSaprs to deal with email requests from the RF side.

The rest of the NOSaprs commands

There are a few NOS commands that do not start with the word ‘aprs’.

- [start|stop] 14501

This is used to start or stop the 14501 HTML status page service.

1.1	[start|stop] 44825

This is used to start or stop the APRS client connection server.

The APRS Client Connection (44825) Server

A few words regarding the 44825 server. I started writing a server in a previous release of NOSaprs, that would allow APRS clients like UI-View and Winaprs to connect to NOSaprs and feed information to it, without getting the firehose of data back (sort of a send only connection). This was ideal for weather stations for example or those clients that had extremely limited bandwidth and only wanted to get their position out to the world and nothing more. It did work, however it ‘seemed’ to have some problems which I never was able to track down on my system. Also, I never liked the original design, so I decided to just work on a new design, which I hope to make available in a future release.

If you want to try the server out, then you will have to edit the ‘aprs.h’ header file in the source code, and recompile the APRS object files, then relink NOS. Look for the 44825 compiler directive. It’s not perfect, but it may very well work for you without any problems.

Note : if you have the 44825 server code compiled into NOSaprs, then additional information about client connections will appear in the 14501 HTML status page as well as in the finger information.

Miscellaneous stuff and other notes

Although APRS was meant for mobile applications, there’s no reason why those people running xNOS for their end user stations, could not configure a simple beacon on their NOS systems. For example, Bob, ve3tok, configured the following beacon in his JNOS, using the NOS ‘ax25 bctext’ command :

ax25 bctext “!4313.13N/07955.25W- VE3TOK TCP/IP 44.135.85.10 Hamilton, Ontario”

If you visit the website, http://www.findu.com, this is how his station will appear :

Here’s a screen shot below, of how your NOSaprs server itself might show up on the findu.com site.

That's all the documentation for now. Please let me know if you see any mistakes or ommissions.

Thank you – have fun using TNOSaprs, and the JNOS version, which I refer to as JNOSaprs.

73 de Maiko Langelaar / VE4KLM