LO/L2 Instructions for alignment of the Kenwood trunk portables and mobiles.

Use the Fleetcall L2.net and Q-trunk L2.net Network files to program the trunk radios.

When setting up customer information in "Personalisation" menu setup the following as well:

Bottom right corner of personalisation menu is a button marked: RSSI Setting. Click on this button.

Next to LO type in a value of -110

Next to L2 type in a value of -70

Handheld: To save battery power switch the "TX power in trunked mode" function on the Trunked feature selection to "AUTO" The handheld will then have a extra setting at the LO/L2 setting selection "Power Threshold (dBm)"  Set this to

-80dBm.

Close window.

Setup rest of customer info and program the radio.

After the radio has been programmed do the following:

Open test mode

Determine the 12dB sinad level of the radio.

Select  RSSI LOW menu. With a signal generator inject the determined 12 dB sinad level into the radio ( + -117 dBm). Select OK.

Open RSSI HIGH menu. With a signal generator inject a signal of -70dBm into the receiver. Click on OK

Close test mode.